Technical assessment for

refrigerant management

Sector: Materials

Agency Level: Business

Keywords: Emissions avoided, end-of-life disposal, refrigerant recovery and destruction, Refrigerant REPLACEMENT, REFRIGERANT banks, gwp

Version 2 (June 2019)

Prepared by:

KAPILNARULA, RESEARCH FELLOW

Marzieh Jafary, Research Fellow

Barbara Rodriguez, Research Fellow

Kevin Bayuk, Senior Fellow

info@drawdown.org             www.drawdown.org

Table of Contents

List of Figures	IV

List of Tables	IV

Acronyms and Symbols Used	VII

Executive Summary	VIII

5	Literature Review	1

1.1	State of Refrigerant Management	1

1.1.1	Refrigerants and Climate Change	1

1.1.2	Refrigerant demand and emissions	7

1.1.3	Reducing refrigerant emissions	9

1.1.4	Commonly Used Refrigerants in Air-to-Air Air Conditioners and Heat Pumps	10

1.1.5	Refrigerant management and conservation	13

1.1.6	Benefits of refrigerant recovery	14

1.1.7	Factors influencing decision to recycle, reclaim or destroy after refrigerant recovery	17

1.1.8	Refrigerant recovery and destruction- technologies and process	17

1.1.9	Replacing refrigerants with low GWP HFC/new cooling agents/non-HFC substances.	23

1.2	Adoption Path	24

1.2.1	Current Adoption	24

1.2.2	Trends to Accelerate Adoption	27

1.2.3	Barriers to Adoption	29

1.3	Advantages and disadvantages of Refrigerant Management	32

1.3.1	Additional Benefits and Burdens	34

2	Methodology	35

2.1	Introduction	35

2.2	Data Sources	36

2.3	Total Addressable Market	40

2.4	Adoption Scenarios	40

2.4.1	Reference Case / Current Adoption	41

2.4.2	Project Drawdown Scenarios	42

2.5	Inputs	47

2.5.1	Climate Inputs	47

2.5.2	Financial Inputs	49

2.6	Assumptions	51

2.7	Integration	52

2.8	Limitations/Further Development	53

3	Results	54

3.1	Adoption	54

3.2	Climate Impacts	55

3.3	Financial Impacts	57

4	Discussion	58

5	References	60

6	Glossary	64

# List of Figures

Figure 1:Environmental Impact of F-gases (Source: Metz et al., 2005)	4

Figure 2: Banks of refrigerants in 2002, BAU-2015 and Mitigation-2015 scenarios (Adapted from Metz et al., 2005)	5

Figure 3: Refrigerant emissions by sector in 2010 Source: (US EPA 2016)	8

Figure 4: Refrigerant demand and emissions by region in 2006 (Source: Denis Clodic, Stephanie Barrault, & Sabine Saba, 2010)	8

Figure 5: Life cycle of refrigerants (Source: Zhao, Zeng, & Yuan, 2015)	9

Figure 6: Several frequently used refrigerants in Water Heating Heat Pumps	12

Figure 7: Modeled Refrigerant Management Solutions - Good Management Practices and Recovery and Destruction at EOL	14

Figure 8: Emissions avoided through implementation of RAD program in the US (Source: USEPA, 2009)	15

Figure 9: Basic refrigerant recovery unit (Source: UNEP, 2010)	18

Figure 10: Refrigerant recovery from a domestic refrigerator (Source: UNEP, 2010)	18

Figure 11: Refrigerant destruction process (adapted from World Bank, 2010)	20

Figure 12: Refrigerants recovered under Responsible Appliance Disposal Program (Source: USEPA )	21

Figure 13: Residential life cycle A/C cost breakdown example (Goetzler et al. 2016)	31

Figure 14: Linkages between recovery, recycling/reusing, reclamation and destruction (Source: Linde AG, n.d.)	33

Figure 15: HFC emissions (GtCO2-eq yr−1) from 2000-2050 (Data source: Velders et al., 2015)	37

Figure 16: Estimated emissions of HFC (only) from various sub-sectors (Data source: Purohit and Isaksson, 2016)	38

Figure 17: Estimated refrigerant banks from 2000-2050 (Source: Velders et al., 2017)	39

Figure 18: World Annual Adoption 2015-2060 HFC Replacement	55

Figure 19: World Annual Greenhouse Gas Emissions Reduction	57

Figure 20: Financial Impacts	57

# List of Tables

Table 1: Phase out schedule of CFCs and HCFCs as per the Montreal Protocol	2

Table 2:  Phase out schedule of HFCs as per the Kigali Accord (WMO 2018)	3

Table 3: ODP, Atmospheric Lifetime, and 100-year GWP Comparison of Common Refrigerants (Washington State University Energy Program and Livingston Energy Innovations 2017)	5

Table 4: Estimated global emissions from F gases in 2005 (Data Source: Gschrey and Schwarz, 2009)	7

Table 5: Summary of results and conclusions of studies on emission reduction	25

Table 6: Core areas of financial and technical assistance by MLF	28

Table 7: Forecast of emissions and refrigerant banks in 2050 (HFC only) (Data source: Gschrey and Schwarz, 2009)	37

Table 8: Estimated HFC refrigerant banks from 2010 to 2050 (Gton CO2 eq.) (Data source: Velders et al., 2015)	41

Table 9: Assumed recovery factors at EOL for various sub-sectors (as % of total banks)	41

Table 10: Estimated refrigerant banks retiring at EOL (ktons)	43

Table 11: Retirement rate of banks (Data source: Purohit and Isaksson, 2016)	43

Table 12: Estimated EOL recovery factorsPDS case (%) (Data source: Purohit and Isaksson, 2016 for HFC; Average of all sub-sectors for CFC and HCFC)	43

Table 13: Estimated recovery of CFC, HCFC ad HFCs at EOL from 2010 to 2050 (ktons)	44

Table 14: Assumed destruction rates in recovered refrigerants (% of total recovered)	44

Table 15: Estimated emissions as leakage from banks (kton)	45

Table 16: Assumed leakage factors for different sub-sectors (as % of total banks)	46

Table 17: Estimated 'in use’ removal efficiency factors PDS case (%) (Data source: Purohit and Isaksson, 2016 for 2040 and beyond; others: linear interpolation)	46

Table 18: TEAP approved destruction technologies and energy consumption	48

Table 19: Climate Inputs	49

Table 20: Million Euro/yr per kiloton Refrigerant by Sub-Sector Data Source: (Purohit &Isaksson,2016)	50

Table 21: Costs for removal and recovery in 2009 (Data Source: TEAP, 2009)	50

Table 22: World Adoption of the Solution	54

Table 23: Climate Impacts	55

Table 24: Impacts on Atmospheric Concentrations of CO2-eq	56

Table 25: Estimated destruction of refrigerant (kton)	58

Table 26: Estimated refrigerant emissions avoided by adoption of good practices (kton)	59

Table 27: Recovery efficiencies at EOL for commercial refrigeration systems	59

# Acronyms and Symbols Used

- GWP – Global Warming Potential

- CFC  –  Cholorfluorocarbons

- HCFC – Hydrocholorfluorocarbons

- HFC – Hydroflurocarbons

- ODS -- Ozone Depleting Substances

- ODP -- Ozone Depleting Potential

- EOL – End of Life of equipment.

- RRD – Removal, Recovery and Destruction

# Executive Summary

Refrigerants are used as working fluid in commercial refrigeration systems, in household appliances such as air conditioners and refrigerators, in refrigerated containers used for carrying perishable goods, air conditioning systems onboard cars, trains, aircrafts and ships and in industrial cooling systems and more.  Most refrigerants in use today have a high Global Warming Potential (GWP) and release of these gasses into the atmosphere contributes to global warming. “Refrigerant banks’’ refer to the total quantity of refrigerants contained in existing equipment like refrigeration and air conditioning appliances, chemical stockpiles, foams etc. which have not yet been released into the environment. Refrigerants are emitted during the production process, from refrigerant banks due to leakages and during end of life (EOL) disposal of appliances and systems. Reducing refrigerant emissions into the atmosphere is the solution Refrigerant Management, which can be undertaken in five main ways:

- Lowering demand/use of appliances and thereby production of refrigerants.

- Replacing refrigerants with low GWP HFC/new cooling agents/non-HFC substances.

- Increasing efficiency of refrigeration in appliances thereby lowering the use of refrigerants.

- Controlling leakages of refrigerants from existing appliances by good management practices.

- Ensuring recovery, reclaiming/recycling and destruction of refrigerants at EOL.

This report and solution models. (ii) specifically, the implications of the recent Kigali accord (iv) and (v). The modeled and measured solution is restricted to refrigeration, heat pumps and AC applications and considers CFC, HCFC and HFC gases separately as these have different GWP and phase out schedules.

Firstly, the model considers the gradual replacement of HFCs by low GWP refrigerants based on peer-reviewed global prognostications.  As HFCs get replaced at the point of production the banks of HFCs decline compared to a reference scenario.  The resulting HFC banks modeled with the adoption of alternative refrigerants are then integrated into a secondary model that measures the potential impact of good management practices and EOL recovery and destruction.

Good management practices is modeled by using the projections available in literature for the size of refrigerant banks and estimates the leakages from these banks using historical leakage rates for various sub-sectors such as commercial, industrial, domestic and stationary AC systems. The adoption of good management practices to control leakages in use considers the technically feasible and plausible improvements in, and accompanying costs of, removal efficiency to quantify the emissions that can be avoided.

The final part of the solution considers the destruction of refrigerants at EOL. The current quantity of refrigerants which are recovered and destroyed at EOL are estimated by using the historical data on equipment retirement rates, recovery efficiency and destruction efficiency. For 2014, this value is estimated to be around 30 ktons of refrigerant which is approximately 2.7% of the total quantity of refrigerants emitted.  This solution models the potential of capturing a higher portion of this refrigerant stream. Estimates of refrigerant banks in the future, less the calculated implications of replacing HFCs with low GWP refrigerants, are used to calculate the EOL banks which retire every year based on the expected lifetime of the equipment. Thereafter higher recovery efficiencies are modeled based on the technical feasibility of recovery from various sub-sectors. Assumptions on increased destruction efficiencies are then used to estimate the refrigerant which can be destroyed in the future along with the cost implications.

All parts of the solution are then combined to calculate the total impact of the solution. Results indicate a maximum annual emission reduction of 8.35 Gt CO2 eq./yr and a cumulative emission reduction of approximately 101.28 Gt CO2eq. from 2020-2050. The solution lifetime operating costs from 2020-2050 are calculated as approximately $610.67 Billion for implementing the solution.

Refrigerant management is difficult to implement as the appliances are distributed and due to limited regulations on controlling leakage of refrigerants, EOL recovery and destruction of refrigerants. Funding, training, technical and informational barriers are also some of the limitations for adoption of the solution.  Policies and regulations on recycling/management of refrigerants are being formulated and implemented. Economic incentives for recovery, recycling and destruction of refrigerants such as issue of carbon credits under the Kyoto protocol would help increase the adoption in developing countries. Capacity building in these countries including technology transfer would help aid faster adoption of the solution.

# Literature Review

## State of Refrigerant Management

### Refrigerants and Climate Change

The American Society of Heating, Refrigerating and Air-Conditioning Engineers (ASHRAE) defines refrigerants as ‘’working fluids in refrigeration, air-conditioning and heat pumping systems, that absorb heat from one area and reject it into another through evaporation and condensation respectively’’ (ASHRAE 2017). A refrigerant absorbs heat at a relatively low temperature in the evaporator (for example, the cooling coil in an air conditioner) and releases it at a higher temperature in the condenser (for example, the outside coil)

Refrigerants are used in commercial refrigeration systems, in household appliances such as air conditioners and refrigerators, in refrigerated containers used for carrying perishable goods, as air conditioning systems onboard cars, trains, aircrafts and ships and in industrial cooling systems etc.

The earliest refrigerants used in large cooling applications were ammonia and carbon dioxide. For small refrigerators in domestic and commercial uses, Sulfur dioxide (SO2) and methyl chloride were used. Owing to the toxicity and flammability of some of these earlier refrigerants, halocarbons were invented and Chlorofluorocarbons (CFCs) like R-11, R-12 and R-502 began to be used as refrigerants. When the ozone hole over Antarctica was discovered, CFCs were identified as causative agents for its formation. CFCs and other halocarbons were labeled as Ozone Depleting Substances (ODS) and countries began to realize the importance of curbing their production and consumption.

The Montreal Protocol signed in 1987, called for a global ban on the production and consumption of ODS including CFCs by January 1, 1996 for developed countries (also called article 2 or non-article 5 countries) and by January 1, 2010 for developing countries (also called article 5 counties). Hydrochlorofluorocarbons (HCFCs) having low Ozone Depleting Potential (ODP) were proposed as ‘’transitional substitutes’’ for CFCs until the time new ozone-safe alternatives could be commercialized (Velders et al. 2009). HCFC-22 became the refrigerant of choice for refrigeration and air conditioning applications. Despite having lower ODP than CFCs, HCFCs still contained ozone destroying Chlorine and HFC-23, a chemical with high GWP which is produced as a result of HCFC-22 production.

In 1992, an amendment to the Montreal Protocol introduced a phase out schedule for production and consumption of HCFCs for both non-article 5 and article 5 countries by certain dates. Further, in another meeting in 2007, a more aggressive phasedown of HCFCs for all countries was agreed to. The phase out schedule of CFCs and HCFCs is described in Table 1.

Table 1: Phase out schedule of CFCs and HCFCs as per the Montreal Protocol

| Ozone depleting refrigerant class |  |  |
| --- | --- | --- |
|  | Non-article 5 countries | Article 5 countries |
| CFCs | January 1, 1996: full phaseout | January 2010: full phaseout |
| HCFCs | January 1, 1996: Freeze at baseline both production and consumption 2.8% of the country’s CFC’s consumption+ 100% of that country’s HCFC consumption | January 1, 2013: Freeze at baseline- average of 2009 and 2010 consumption and production |
|  | January 1, 2004: cut by 35% | January 1, 2015: cut by 10% |
|  | January 1, 2010: cut by 75% | January 1, 2020: cut by 35% |
|  | January 1, 2015: cut by 90% | January 1, 2025: cut by 67.5% |
|  | January 1, 2020: cut by 99.5% (can be used only in servicing of refrigerators or ACs after this date) | January 1, 2030: cut by 97.5% (can be used only in servicing of refrigerators or ACs after this date) |
|  | January 1, 2030: full phase out | January 1, 2040: full phase out |

HCFCs were replaced with HFCs such as R404A, R134a, R 407 etc. because of their low ODP. HFCs however have a significant GWP and fall under the group of gases being controlled by the Kyoto Protocol.

HFCs are gradually being replaced by Hydrocarbons or HCs. In a further boost to the phasing out of HFCs, countries have agreed to an amendment of the Montreal Protocol in Oct 2016, which is dubbed as the Kigali accord. Table 2 shows the summary of the phase out schedule for different countries. The detailed year wise phase out schedule can be accessed from the UNEP website.

Table 2:  Phase out schedule of HFCs as per the Kigali Accord (WMO 2018)

|  | Non-article 5 countries (Developed Countries.) | Article 5 countries Group 1 | Article 5 countries Group 2 |
| --- | --- | --- | --- |
| Base Level | Average HFCs 2011-2013 plus 15% of HCFC base level | Average HFCs 2020-2022 plus 65% of HCFC base level | Average HFCs 2024-2026 plus 65% of HCFC base level |
| Freeze | - | 2024 | 2028 |
| 1st step | 2019: 10% reduction | 2029: 10% reduction | 2032: 10% reduction |
| 2nd step | 2024: 40% reduction | 2035: 30% reduction | 2037: 20% reduction |
| 3rd Step | 2040: 50% reduction | 2040: 50% reduction | 2042: 30% reduction |
| 4th Step | - | - | - |
| Plateau: | 2036: 85% reduction | - 2045: 80% reduction | - 2047: 85% reduction |

It is claimed that the legally binding expansion to the treaty that protects the ozone layer can avoid up to 0.4°C of global warming this century by reducing the use of powerful climate-warming refrigerants known as hydrofluorocarbons by 80 percent (UNEP 2018),

“Refrigerant banks’’ generally refers to the total quantity of refrigerants contained in existing equipment like refrigeration and air conditioning appliances, chemical stockpiles, foams etc. which have not been released into the atmosphere. Refrigerants are also produced every year for charging of new appliances and are used for recharging into existing appliances. Additional refrigerants which are produced add to the stock of refringent which already exist in old appliances, thereby increasing the size of refrigerant banks.

Figure 1 illustrates the contribution of refrigerant gases to ozone hole and climate change. Emissions from refrigerants can occur during production, as leakages from refrigerant banks (from servicing, during recharging) and from venting during end of life disposal of equipment. Despite the global transition to low-GWP and zero-ODP refrigerants, there still exist vast quantities of refrigerant banks (CFCs and HCFCs). Refrigerant banks are growing as the production rate of equipment is higher than the retirement rate of equipment leading to increased banking of refrigerants. This is due to increasing use of AC and refrigeration systems in emerging economies which is directly proportional to economic prosperity. It is estimated that refrigerant banks are about 5-10 times the annual production of refrigerants. The banks of HFCs, in particular, are growing annually. Hence the potential emissions from these banks present a multi decade commitment to climate change and if these are not managed, they could contribute significantly to global warming in the future. That being said, under Kigali amendment that entered into force on 1 January 2019, the article 5 and non-article 5 countries will be committed to phase-down their HFC consumption.

Figure 1:Environmental Impact of F-gases (Source: Metz et al., 2005)

Figure 2: Banks of refrigerants in 2002, BAU-2015 and Mitigation-2015 scenarios (Adapted from Metz et al., 2005)

In 2002, the Intergovernmental Panel on Climate Change (IPCC) and the United Nations Environment Program’s Technology and Economic Assessment Panel (TEAP) estimated banks of CFCs, HCFCs, HFCs in refrigeration.  Figure 2 shows the refrigerant banks in 2002 and the estimated banks forecasted for 2015. These were projected to be about 4,751 Mtonnes CO2eq. (GWP weighted), projected to decrease to 4,157 Mtonnes CO2eq. in 2015 under a BAU scenario and 3548 Mtonnes CO2eq. under a mitigation scenario (IPCC-TEAP Metz et al. 2005). The mitigation scenario assumed global application of best practices for the use, recovery and destruction of refrigerants; 60% of these reductions would come from HFCs and the remaining from HCFCs (30%) and CFCs (10%).

A summary of common refrigerants with their ODP, ODP, Atmospheric Lifetime and 100-year GWP are presented in Table 3.

Table 3: ODP, Atmospheric Lifetime, and 100-year GWP Comparison of Common Refrigerants (Washington State University Energy Program and Livingston Energy Innovations 2017)

| Refrigerant | ODP | Atmospheric lifetime (years) | GWP (100 years) | Safety group | Notes |
| --- | --- | --- | --- | --- | --- |
| CFC-11 | 1 | 50 | 400 | A1 |  |
| CFC-12 | 1 | 102 | 8,500 | A1 |  |
| CFC-112 | 0.8 | 85 | 5,000 |  |  |
| CFC-114 | 1 | 300 | 9,300 | A1 |  |
| CFC-115 | 0.6 | 1700 | 7,300 | A1 |  |
| HCFC-22 | 0.55 | 13.3 | 1,700 | A1 |  |
| HCFC-123 | 0.02 | 1.4 | 93 | B1 |  |
| HCFC-124 | 0.022 | 5.9 | 480 | A1 |  |
| HCFC-141b | 0.11 | 9.4 | 630 | A2 |  |
| HCFC-142b | 0.065 | 19.5 | 2,000 | A2 |  |
| HCFC-225c/a | 0.025 | 2.5 | 170 |  |  |
| HFC-23 | 0 | 270 | 14,800 | A1 |  |
| HFC-32 | 0 | 5.6 | 650 | A2 |  |
| HFC-125 | 0 | 32.6 | 2,800 | A1 |  |
| HFC-134a | 0 | 14.6 | 1,430 | A1 |  |
| HFC-143a | 0 | 48.3 | 3,800 | A2 |  |
| HFC-152a | 0 | 1.5 | 140 | A2 |  |
| HFC-404a | 0 | 40.4 | 3,260 | A1 | Refrigerant blend |
| HFC-407c | 0 | 15.7 | 1,530 | A1 | Refrigerant blend |
| HFC-410a | 0 | 17 | 1,730 | A1 | Refrigerant blend |
| HFC-434a | 0 | 30 | 3,245 |  | Refrigerant blend |
| HFC-437a | 0 | 17 | 1,805 |  | Refrigerant blend |
| HFC-507a | 0 | 40.5 | 3,300 | A1 | Refrigerant blend |
| HFO-1234yf | 0 | 0.029 | 1 | A1 |  |
| HFO-1234ze | 0 | 0.045 | 1 | A2L |  |
| HFO-1336mzz | 0 | 0.06 | 2 | A1 |  |
| R-170 | 0 | 12 | 6 | A3 | Ethane |
| R-290 | 0 | 12 | 4 | A3 | Propane |
| R-600 | 0 | 12 | 5 | A3 | Butane |
| R-600a | 0 | 12 | 5 | A3 | Isobutane |
| R-601 | 0 | 12 | 11 |  | Pentane |
| R-717 | 0 | 0.03 | 0 | B2 | Ammonia |
| R-744 | 0 | 100 | 1 | A1 | CO2 |

### Refrigerant demand and emissions

In order to measure and model the potential impact of refrigeration management it is important to understand the complexity and variability of demand for and sources of emissions related to refrigerants.

Table 4 shows the estimated global emissions from use of refrigerants (referred to collectively as F gases) in 2005. It is estimated that 86% of the emissions were from the refrigeration and air conditioning (Ref. and AC) subsector and the remining were contributed by foam, Metered Dose Inhalers (MDI), aerosols and solvents, firefighting and other applications.

Table 4: Estimated global emissions from F gases in 2005 (Data Source: Gschrey and Schwarz, 2009)

|  | Ref. and AC | Foam | MDI | Aerosols and solvents | Fire fighting | Total |
| --- | --- | --- | --- | --- | --- | --- |
| MTonnes CO2 eq. | 1679 | 143 | 58 | 32 | 36 | 1960 |
| Share of subsectors | 86% | 7% | 3% | 2% | 2% | 100% |

It is seen that the refrigeration and AC subsector contributed to 86% of the global emissions. By limiting the scope of the research and analysis of this solution to look only at the refrigeration and AC sub-sector and is thereby likely to account for around 85% of F gases.

Sector-wise use

Figure 3 shows the refrigerant emissions in different 2010 in various sectors. The major contributor to emissions is the electricity and heat production followed by agriculture, forestry and other land use and the industry subsector.

Figure 3: Refrigerant emissions by sector in 2010 Source: (US EPA 2016)

Region-wise use

Figure 4 shows the refrigerant demand and emissions at the regional level in 2006. Refrigerant demand in U.S. was highest globally, closely followed by China, EU-27 and then South and East Asia.

Figure 4: Refrigerant demand and emissions by region in 2006 (Source: Denis Clodic, Stephanie Barrault, & Sabine Saba, 2010)

### Reducing refrigerant emissions

Figure 5 illustrates the life cycle of refrigerants.

Figure 5: Life cycle of refrigerants (Source: Zhao, Zeng, & Yuan, 2015)

Direct GHG emissions occur during production, filling or refrigerant charging, servicing and disposal. Indirect emissions from energy consumption during these processes also add to the GHG emissions. According to a study assessing the carbon footprint of refrigerants, 90% of the direct GHG emissions of high and middle GWP refrigerants are due to the disposal process (Zhao, Zeng, and Yuan 2015). Majority of the global effort in reducing emissions from refrigerants has focused on the following areas:

- Change or shift to alternative low GWP or zero GWP refrigerants

- Improving the overall efficiency of refrigerators through technology improvements

- Better servicing and maintenance of existing refrigeration equipment

These efforts have been supported by the Montreal Protocol and by national regulations and policies (e.g., the Ecodesign directive for improving refrigerator efficiency and the F-gas regulations for mandating improved service practices).

While it is easier to quantify and project the emissions reduction potential due to interventions like use of alternative climate-friendly refrigerants, efficiency improvements, adoption of refrigerant management practices like improved installation, charging, servicing and end of life disposal by modelling the assumptions, getting actual data on these practices is much more difficult. Data on these practices is not available on a global scale. Hence, assumptions have been made in this analysis for quantifying various variables to assess the potential of reducing emissions from refrigerant management. Refrigerant recovery and good management practices are estimated to have a significant potential to reduce GHG emissions from refrigerants, for example a 20% reduction in direct emissions from refrigerants was reported for a 50% refrigerant recovery in ACs (Zhao, Zeng, and Yuan 2015).

### Commonly Used Refrigerants in Air-to-Air Air Conditioners and Heat Pumps

As mentioned in the history of Fluor Chemicals, HCFC-22 (R-22) has been the most popular refrigerant for residential heat pump and air-conditioning systems for over four decades. Unfortunately, the system leakage contributes to ozone depletion. It is also a greenhouse gas, and its manufacture yields a by-product HFC-23 that in turn contributes to global warming. As of 2010, EPA banned the production, import, and use of HCFC-22 and HCFC-142b, except for continuing servicing needs of existing equipment, achieving 75% of reduction goal (EPA 2016).

Air conditioner manufacturers are now offering alternatives to HCFC-22 that use non-ozone depleting refrigerants. For now, existing HVAC units using HCFC-22 can continue to be serviced with it. After 2020, the servicing of R-22-based systems will rely solely on recycled or reclaimed refrigerants. It is expected that this action will ensure its existing supplies will last longer and be available to serve a greater number of systems still using this refrigerant until complete phase-out on January 1, 2030. For air-to-air air conditioners and heat pumps, the most substantial recent developments are related to the increased rate of substitution of HCFC-22 and the greater consideration of the use of medium and low GWP alternatives. Some manufacturers are adopting HCs, and there is also uptake of HFC-32 (UNEP, 2016). HCFC-22 is still widely used in new and existing systems in Article five (developing) countries and to some extent in existing systems in non-Article five (developed) countries.

The R-400 series is made up of hydro-fluorocarbon (HFC) zeotropic blends. The most popular is R-410A, which does not contribute to depletion of the ozone layer; rather, similar to R-22, it contributes to global warming. R-410A is the most common refrigerant for new light commercial A/C systems, small to medium-sized HP, and HPWH, but operating pressures are more than 50% higher than R-22. Consequently, R-410A systems require components capable of working at these higher pressures. The transition away from ozone-depleting R-22 to systems that rely on replacement refrigerants as R-410A has also required a redesign of the heat pump and A/C systems. New systems incorporate compressors and other components specifically designed for use with these replacement refrigerants. R410a is supercritical above a temperature of 71 °C. Above this temperature, a difference between the gas and liquid phase can no longer be made. R410a can be applied in low-temperature heat pump systems. As compared to R134a the volume that can be swept by the compressor is smaller; implying lower investment costs. On the other hand: its efficiency as compared to R134a is lower as well.

The majority of new systems using an alternative to HCFC-22 utilize R-410A; while some others are using R-407C, HFC-134a, HC-290, and HFC-32 (Figure 2). Modern refrigerators and as a refrigeration in medium-sized or large HPWH usually use HFC-134a that does not contribute to the depletion of the ozone layer. The pressure in R134 is fairly low. Due to this, the volume that needs to be swept by the compressor is rather high and therefore higher investments are needed for installation.

R32 is a single component, zero ODP gas with a GWP of 550, significantly less than R410A's 1,980. It is classified as A2, or mildly flammable, under ASHRAE classifications which has the potential to add design complexity in order to ensure the safety of technicians and building occupants.

Ironically, R410A is a mixture of R32 and R125 – where the R125 is suppressing R32's flammability. That being said, HFC-32 have been commercialized in the residential air conditioners in Japan, Taiwan, the Philippines, Indonesia, Vietnam, Thailand, Malaysia, Australia, India, Singapore, New Zealand, 28 countries in Europe, Ukraine, Russia, Norway, Montenegro, Albania, Turkey, UAE, Saudi Arabia, Oman, and Sri Lanka as of the end of March 2016 (Daikin, 2016). On the other hand, there are a growing number of alternatives with a medium to low GWP and are flammable that are being considered and evaluated by research entities and enterprises; meaning there is some degree of uncertainty over the future selection of alternatives.

The most popular of the R-600 series refrigerants is R-600a, which is also called isobutane. R-600a, coming from natural sources, has zero ozone depletion potential and negligible global warming capacity. The biggest downfall is that isobutane is flammable and classified as A3 under ASHRAE standards, therefore systems must be designed with that in mind and technicians must be trained to handle it properly. R-600a is most commonly used in domestic refrigerators in Europe (primarily in Germany), but it has been used in commercial refrigerators in the United States, albeit in very limited numbers.

Figure 6: Several frequently used refrigerants in Water Heating Heat Pumps

For space-heating and water-heating heat pumps, legislation on minimum energy efficiency has entered into force in Europe, Japan, and the USA, and has decreased the number of air to water heat pumps that can be placed on the market (UNEP, 2016). For water-heating heat pumps most systems commercialized today make use of R-410A, HFC-134a, R-407C, HC-290, HC-600a, R-717 or R-744. The majority of new equipment uses R-410A. In some Article 5 countries, HCFC-22 is used. There are no technical barriers to replacing HCFC-22 with a non-ODS refrigerant in new systems. The main parameters in the selection of alternatives when switching over from HCFC-22 are efficiency, cost-effectiveness, economic impact, safe use and ease of use. R-744-based water-heating heat pumps have been mainly developed and commercialized in Japan. However, the expansion of this technology to other countries is limited due to its high cost. R-717 has also been used in a small number of reversible and absorption heat pumps.

R-407C has been used in small to medium-sized HP & AC. These refrigerants are regularly used in installations that have the dual function of both air conditioning and heating.

Although EPA’s lists of alternative refrigerants urges that R290 (propane) and R600a (isobutane) are among unacceptable refrigerants as of 3 January 2017; however, the research on the performance of these two refrigerants as the substitute of R134a for heat pump water heating indicated that both alternative refrigerants could provide comparable system performance to that of the baseline system containing R134a, with one caveat. As a drop-in alternative, R290 was found to be a better substitute for R134a since it has higher COP, whereas R600a is expected to provide similar performance if the compressor size is increased to provide similar heating capacity. Significant reductions in system charge and lower condenser discharge temperatures were identified as additional benefits (Nawaz et al. 2017).

### Refrigerant management and conservation

United Nations Environment Program (UNEP) defines refrigerant management as an approach to optimize the use of available refrigerants in the existing equipment and minimizing the demand for virgin refrigerants for servicing through technical and regulatory measures (UNEP 2010). The objective of refrigerant management is to reduce the overall harmful impact of refrigerant emissions on the environment throughout the refrigerant lifecycle. Refrigerant conservation is a part of refrigerant management and involves practices for containing, recovery, recycling and reuse of refrigerants. The assessment report of UNEP program’s Refrigeration, Air-conditioning and Heat Pumps Technical Committee or RTOC discusses refrigerant conservation as an important measure for extending the life of refrigeration equipment (RTOC 2011). This can be undertaken through:

- refrigerant recovery, recycle and reuse

- proper disposal or destruction of refrigerant banks in existing stock of appliances and

- eliminating leakages from systems.

The definitions of recovery, recycle, reclaim and destruction as defined by the International Standards Organization or ISO (ISO 1999) are given below:

- Recover: to remove refrigerant in any condition from the refrigerator and store it in an external container. Refrigerant recovery for small domestic refrigerators can occur on-site by a certified/trained professional. For larger units, transportation to a R&R (Recovery & Recycling) facility will be required

- Recycle: to clean the recovered refrigerant using oil separation and filtration techniques to make the refrigerant suitable for reuse. This takes place in a recycling facility

- Reclaim: It is the reprocessing of used refrigerants to virgin product specifications. Contaminants are removed, and a chemical analysis is done to determine if the appropriate specifications are met. Reclamation occurs at a reprocessing or manufacturing facility

- Destroy: To transform the used refrigerant into other chemicals in an environmentally responsible manner.

This report and corresponding model assess refrigerant recovery and destruction as one of the approaches under refrigerant management that can contribute to emissions reductions. It also considers the potential of reduction of refrigerants by eliminating leakages from the refrigerant banks (appliances) and replacing of HFCs with low GWP refrigerants.

Figure 7: Modeled Refrigerant Management Solutions - Good Management Practices and Recovery and Destruction at EOL

### Benefits of refrigerant recovery

The IPCC-TEAP’s special report on ODSs estimated that with a typical lifetime of 20 years, EOL disposal of 5% of the installed base, 75 million refrigerators containing approximately 7500 million tons of refrigerant are retired and disposed of annually (Metz et al. 2005). The same study also predicted that the total direct emissions from the banks in refrigeration and air conditioning could reach 2.3Gt CO2 eq per year by 2015 which would cancel all reductions achieved under the Kyoto Protocol. According to EIA, recovery and destruction of these banks will have an added benefit of reducing the costs of recovery and destroying the remaining 14 Gt CO2 eq. in banks that will be emitted after 2015 (EIA 2009). Thus, refrigerant recovery followed by additional practices of recycling, reclamation and destruction presents enormous environmental and economic benefits.

Environmental benefits

GHG reduction benefits

According to IPCC-TEAP, recovery, recycling and reclamation, and the destruction of refrigerants not suitable for reprocessing, may reduce emissions by as much as 20% by cutting down on refrigerant demand due to reclaimed refrigerant (Metz et al. 2005). The recycling and reuse of refrigerants can reduce the direct emissions from refrigerant disposal and production of new refrigerants including indirect emissions from energy consumption during production. The amount of recovered refrigerant recycled, reclaimed or destroyed translates to an equivalent reduction in refrigerant demand and corresponding emissions. USEPA (2009) estimated the climate benefits of the recovery of CFCs, HCFCs and HFC refrigerants and foam blowing agents from household appliances through the Responsible Appliance Disposal (RAD) program. Figure 8 describes the total GHG emissions avoided through RAD program in the US.

Figure 8: Emissions avoided through implementation of RAD program in the US (Source: USEPA, 2009)

While EPA administers a voluntary program for household appliances called the Responsible Appliance Disposal (RAD) Program, due to decreasing participation emissions reductions have declined from a height of 2.2 million MTCO2 e in 2012 to just 0.7 million MTCO2 e in 2016 of refrigerant emissions avoided.(Source) The RAD Program is a voluntary partnership with private sector utilities and manufacturers and states to properly recover and dispose of refrigerant and foams contained in old household refrigerated appliances. Since being established in 2006, the RAD program has resulted in recovery of 14 million metric tons of CO2 e of refrigerants and foams. EPA estimates about 9 million refrigerators are thrown away each year.(Source) At the height of its impact, the RAD Program was addressing about 9% of the household refrigeration sector, which declined to about 5.5% in 2016 and participation of states in the RAD program has been very limited.40 The potential emissions reduction from this sector thus remains significantly high.

Materials reduction:

Refrigerant recovery is akin to resource recovery. In a survey conducted by Refrigerant Reclaim Australia (RRA) it was found that refrigerant equivalent of 25% of refrigerant sales can be recovered in any given year. Also, recovery facilities for refrigerants can also be used to extract other gases which are present in foams (which are used in insulation of refrigeration appliances).

Economic benefits:

Meeting refrigerant demands for servicing:

From an economic perspective, it is critical that CFCs and HCFCs banked in existing stock be fully recovered, recycled/reclaimed or destroyed.  Recycled or reclaimed ODS refrigerants can be essential for satisfying the demand for servicing in article 5 countries post-2010 (ICF International 2010). However due care also needs to be given to ‘’unwanted’’ refrigerant gases in article 5 countries that have no market value either because of contamination or from lack of access to reclamation infrastructure. These stocks must be contained properly to minimize leakages and reclaimed or destroyed as capacities and capabilities in these countries are developed. While refrigerant recovery can directly impact refrigerant demand and thus conserve refrigerants, other techniques like preventing leakages, better installation and hygienic servicing practices also conserve refrigerants. Such well-designed and optimally performing systems also consume less energy.

Reduced environmental compliance costs:

For countries where strict regulations for refrigerant recovery exist, proper recovery ensures that compliance costs are minimized.

### Factors influencing decision to recycle, reclaim or destroy after refrigerant recovery

Reclaimed or recycled refrigerants meet existing refrigerant demand for servicing or new production. However, the decision to reclaim, recycle/reuse or destruction depends on several factors. TEAP (2009) has identified the following factors affecting the decision to recycle, reclaim or destroy:

- Purity of recovered refrigerant: Cases where the recovered refrigerant can be used within the same or similar equipment after recycling, reclamation is avoided since it is a costly process of removing contaminants from refrigerants. Reclamation becomes a preferred option when price of reclaimed refrigerant is maintained at a sufficiently high level to encourage reclamation and minimize venting.

- Equipment age and condition: Recycling is most appropriate for equipment with relatively low leak rates.

- Mixture of ODSs or significantly contaminated substance: Reclamation is not an [economic] alternative in regions where there is plentiful supply of virgin material or where there is little refrigerant demand.

- Accelerating technology transition: When regulators have decided to reduce reliance on ODS refrigerants by banning manufacture of new equipment and limiting the amount of refrigerant that can be recycled, recycling is no longer preferred. Mandatory destruction or venting bans can be put in place.

- Pricing: Where the virgin material has the lowest price, it will be used and there will be no demand for recycled material. In this case either venting or destruction of refrigerants will be done. If destruction creates a financial benefit, it would be preferred. When the recycled material has a lower cost than virgin material or virgin material is in short supply, there will be demand for recycling refrigerants. Decision to preferentially destroy will be made only if the profit from destruction is higher than profit from sale of recycled refrigerants. Finally, when there is no market for refrigerants, destruction is the only option to prevent release of refrigerant into the environment.

### Refrigerant recovery and destruction- technologies and process

Refrigerant recovery units

Refrigerant recovery units are becoming increasingly available, lighter and user-friendly. They are available in a wide range of features and prices. According to UNEP’s manual for service technicians in the refrigeration industry (UNEP 2010), the design of the recovery unit i.e. its capacity and rate of recovery varies based on the application, and the type of refrigerant that be recovered. Figure 9 shows the main components and configuration of a recovery unit.

Figure 9: Basic refrigerant recovery unit (Source: UNEP, 2010)

A recovery unit is connected to the refrigerator by valves. After the recovery unit is connected, there are three methods to recover refrigerants. Figure 10 illustrates refrigerant recovery from a domestic refrigerator. According to IPCC-TEAP, the cost for recovery and recovery/recycling units start at approximately $500 (Metz et al. 2005).

Figure 10: Refrigerant recovery from a domestic refrigerator (Source: UNEP, 2010)

There are three methods for refrigerant recovery:

- Vapor transfer: The refrigerant is removed from the refrigerator in a vapor state and condensed into a liquid by the recovery unit and transferred to a recovery cylinder. It is the most common method of recovery and practiced for smaller domestic refrigerators

- Push-pull recovery: This method is used when large volumes of liquid refrigerants need to be recovered. The recovery unit pulls vapor from recovery cylinder and produces high pressure discharge gas that pushes the liquid refrigerant out of the refrigerator and back into the recovery cylinder

- Liquid recovery method: This method has been devised recently and has gradually become the preferred method of recovery. It works just like vapor transfer method and the refrigerant is recovered in liquid state. This is also suitable for refrigerant recovery from large refrigeration systems.

Refrigerant destruction

Figure 11 shows the process of refrigerant destruction. Refrigerant gases are recovered and stored in cylinders. Recovery cylinders from several units are consolidated in and transported to a destruction facility. Prior to destruction, testing is done to verify the contents of the cylinder. Destruction is done using any of the 11 TEAP approved technologies. These technologies are broadly classified into

- incineration

- plasma and

- other non-incineration technologies.

Figure 11: Refrigerant destruction process (adapted from World Bank, 2010)

Here is a summary of policies and programs on refrigerant management and recovery in some non-article 5 countries:

- European Union: In 2006, the European Union (EU) adopted the F-gas regulations that contained a number of provisions to monitor the consumption and reduce the emissions of F-gases (HFCs, PFCs and SF6) including labeling, reporting and leak checking requirements. Article 4 of the regulation also required placing proper arrangements for ensuring the recovery of F-gases at the servicing, maintenance and end of life stages of refrigeration appliances for the purposes of recycling, reclamation or destruction. However, it was observed that the due to the absence of national provisions there was considerable delay in the implementation of the regulations in all member states. The regulation was strengthened further in 2014 when adoption of more stringent norms related to recovery and disposal were introduced. Several EU member states like Netherlands, France and Germany have also enacted national legislations to achieve the goals of the F-gas regulations. It is estimated that due to the new F-gas regulations, the EU’s emissions of F-gases will be cut down by two-thirds by 2030 compared to 2014 levels. Clodic and Barrault (2011) report a reduction in CO2eq refrigerant emissions from 170 MT in 1990 to 147 MT in 2010 despite the doubling of refrigerant bank from 200,000 tonnes in 1990 to 510,000 tonnes due to accelerated phase-out of CFCs and HCFCs and the F-gas regulations.

- United States: Section 608 of the Clean Air Act prohibits venting of ODSs used as refrigerants into the atmosphere during regular maintenance, servicing, repairing services and end of life disposal of refrigeration systems. The USEPA has established certification programs for refrigerant recovery and recycling equipment under which all manufacturers or importers of such equipment must ensure that testing of equipment at EPA-approved testing facilities. Recovery and recycling equipment used for refrigeration systems must be tested as per ARI 740 test protocol. Section 608 also mandates periodic checks on leakages from installed systems and their repair by certified professionals. Recovery, recycling and reclamation can only be performed by EPA-certified technicians.  Additionally, section 609 restricts sale of virgin or reclaimed refrigerants for stationary refrigeration. Refrigerant recovery from domestic refrigeration appliances is also addressed through the voluntary Responsible Appliance Disposal (RAD) program. Figure 12 presents the  result of RAD partners’ efforts to target older, inefficient units. In the coming years, more hydrofluorocarbon (HFC) units will be retired. Proper handling of HFC refrigerants and blowing agents at end-of-life will reduce GHG emissions.

Figure 12: Refrigerants recovered under Responsible Appliance Disposal Program (Source: USEPA )

Federal law requires that all refrigerant be recovered before dismantling or disposal and that universal waste—including mercury—as well as used oil and polychlorinated biphenyls )PCBs( be properly managed and stored. However, because there may be little to no economic incentive for the proper disposal of these materials, their ultimate fate is often unknown. There have been reports of appliance dumping, venting of refrigerant, and release of hazardous components to the environment. Some of these improper disposal practices (i.e., the release of refrigerant and other hazardous components) may still be referred to as “appliance recycling” if the metals or other durable components are recycled (the United States Environmental Protection Agency).

- Australia: In 2003, the Ozone Protection and Synthetic Greenhouse gas management Act was passed by the Australian parliament that made it mandatory for refrigeration and air conditioning industry to ‘’recover, return and safely dispose of ODSs and synthetic GHG refrigerants Recovery and reclamation professionals must be certified by the Australian Refrigeration Council or ARC. The law prohibits venting, requires that sellers of ODSs and GHG refrigerants accept back recovered refrigerants, importers of refrigerants to have valid importers license and companies buying or selling refrigerants must meet specified criteria. There are stiff penalties for non-compliance. An offence under the regulation carries a maximum penalty $1100 for individuals and $5500 for corporate entities.

In Australia, Refrigerant Reclaim Australia (RRA), a not-for-profit industry-funded organization was established that recovers, reclaims and destroys ozone-depleting refrigerants. RRA is strongly supported by key industry participants, involving importers and wholesalers of refrigerants, and relevant industry associations such as equipment manufacturers and contractors. This inclusiveness is considered to be a major factor in the success of the program. The role of RRA is to collect, reclaim, or destroy unwanted and contaminated material. The concept of recycling and reuse of refrigerants is also widely promoted.

- Japan: Japan has two separate laws mandating recovery and destruction of fluorocarbons in the commercial and domestic refrigeration sectors. The fluorocarbons recovery and destruction law, 2001 is applicable for commercial refrigerators and the home appliances recycling law, 1998 covers domestic refrigerators. Both laws place obligations on the end-users to ensure recovery of refrigerants at the end of life of appliances. The recovery operators are required to work with destruction operators for destruction of recovered refrigerants if the recovered refrigerant does not hold any value for the former. As of 2008, 30,000 recovery operators were registered with local governments and 74 destruction operators held licenses from the Ministry of Environment, Japan.

While it is fair to assume that the measures described above have contributed to the success of refrigerant recovery initiatives in these countries, other factors like geography, technology availability/infrastructure, industry support and economics also play an important role:

- Economics: Narrowing down the scope of refrigerant recovery to a few end-use categories where it is economically viable to recover and recycle refrigerants. For example, Japan has a targeted recovery program only for commercial refrigeration systems

- Industry support: In Australia, RRA receives strong support from industry stakeholders like importers, wholesalers of refrigerants, equipment manufacturers and contractors. In 2001, 63% of the refrigerant recovered by RRA was recycled and reused or stored by contractors for later use.

In a survey of the ODS management capacities and capabilities of two article 5 countries (India and Colombia) it was found that environmentally responsible management of refrigerants was still in its nascent stages with initial programs on technician training and disbursing of recovery equipment underway but procedures and regulations for collection, reclamation and destruction of refrigerants were not yet established (ICF International 2008). In these countries, the lack of economic viability of refrigerant recovery is a major obstacle. The MLF, UNEP and UNIDO have supported programs and capacity building initiatives to encourage recovery and recycling of CFCs and HCFCs.

### Replacing refrigerants with low GWP HFC/new cooling agents/non-HFC substances.

In order to achieve a global phase-down of HFCs, replacement fluids must be identified, and a global refrigerant transition must be implemented. Currently, there is no single replacement fluid, all replacements include trade-offs. The theoretically ideal refrigerant is the one having zero ozone depletion potential (ODP), low global warming potential (GWP), nontoxic, non-flammable, has appropriate thermodynamic and heat transfer properties and is compatible with any type of lubricating oil (El-Sayed, El Morsi, and Mahmoud 2018). According to El Sayed et al. Hydrocarbons (HCs) satisfy all the requirements except being highly flammable oil (El-Sayed, El Morsi, and Mahmoud 2018).

Climate-friendly alternatives to HFCs are increasingly available for most uses and fall into two basic categories: “natural” or non-fluorinated substances with low GWPs, and fluorinated substances with low to mid-range GWPs (Borgford-Parnell et al. 2018). Currently, there is an emphasis on natural refrigerants that are gradually being used as a replacement for refrigerants from the hydrofluorocarbon group (Zanchi, Boban, and Soldo 2019). Commercially available  “natural refrigerants” include: ammonia (R-717), with a GWP of near zero; hydrocarbons (HC )(e.g., propane [R-290] and isobutene [R-600a]), with GWPs of less than 4; and CO2 (R-744)  (with a GWP of 1). The use of hydrocarbons (e.g., iso-butane and propane ammonia and carbon dioxide).

Commercially available fluorinated substances include O2, the low-GWP HFCs, also known as hydrofluoro-olefins (HFOs), some with GWPs of less than 1, according to the IPCC. Other alternatives include, HFC-32, with a GWP of 677, and HFC-152a, with a GWP of 138, according to the IPCC. There are also “not-in-kind” alternatives that do not involve refrigerants, such as district cooling. Improving energy efficiency together with the HFC phasedown could approach 80 to 100 billion tonnes avoided from appliance efficiency alone, nearly doubling the climate benefits of HFC phasedown alone (IGSD 2018) .

## Adoption Path

### Current Adoption

Refrigerant Replacement with low GWP Refrigerants

Whereas refrigerant management has been implemented to various degrees in different regions around the world, regulations and incentives are not yet universally adopted, but attempts have been made to assess and measure the potential adoption of various aspects of refrigerant management that provide some insight into the factors which are likely to influence adoption.

Currently, Europe is the leader in terms of legislation for the reduction in HFCs. The Regulation (EU) no. 517/2014 (F-Gas regulation) was implemented in January 1, 2015, putting in place an HFC phase-down from 2015 to 2030 by means of a quota system (Danfoss 2018) and sectorial bans on high GWP refrigerant, with the goal of reducing 79% of  HFC emissions by 2030 compared to the average levels in 2009-2012 (Zanchi, Boban, and Soldo 2019) . This has led to increasing prices for HFCs, and as a result a transition to natural refrigerants (CO2 and hydrocarbons (HCs)) in refrigeration is underway on a wide scale across Europe. As of 2018 50% of HFC has been reduced in Europe, the majority of this reduction comes from supermarkets replacing R404a to CO2 (Danfoss 2018).

As of January 2018, CO2 Transcritical (TC) supermarket stores account for more than 14% of total stores in Europe (Shecco 2018). In 2008 there were only 140 stores using TC in 2008, and this went up to 16,000 stores using TC as of 2018. The next leader in TC technology would be Japan with 3,530, while in the US. TC CO2 stores account for only about 1% of the total market (Shecco 2018). According to Shecco the TC CO2 for industrial refrigeration market is expected to grow five-fold by 2020 (Shecco 2018).

In relation to hydrocarbon (HC), Shecco has estimated that as of August 2018, there were 2.5 million hydrocarbon-based supermarket showcases worldwide.

In relation to low-charge ammonia (NH3), as of  2018, there were: 1440 stores in Europe, 500 stores in Japan, 200 stores in Canada, 200 stores in the US, 150 stores in China and 100 stores in Australia (Shecco 2019).

Good Management Practices and Recovery and Destruction of Refrigerants at EOL

According to TEAP, the global flow of ODS which is estimated to be available for disposal is likely to peak at 200,000 to 225,000 annually within the period of 2018-2020 and 90% of this would be from refrigerants. Few studies have estimated the impact of implementation of refrigerant recovery on emissions reductions globally. Table 5 gives a brief summary of these studies, their assumptions and conclusions:

Table 5: Summary of results and conclusions of studies on emission reduction

| Study/Report | Forecasting Period | Scenarios and assumptions | Results and conclusions |
| --- | --- | --- | --- |
| Clodic and Barrault (2011) | 2006-2030 | Aimed at understanding the impact of F-gas regulations on emissions from refrigeration and air conditioning sector. Models impacts under two scenarios: - F-gas: full implementation of the F-gas Regulation as well as on the introduction of lower GWP refrigerants - F-gas plus: aggressive introduction of lower GWP refrigerants; more stringent efforts on containment, end of life recovery and refrigerant charge reduction | Despite the growing bank of refrigerants, CO2 eq emissions significantly decrease under both F-gas and F-gas plus scenarios: F-gas scenario reductions: From 147 million tonnes in 2010 to 125 million tonnes or 15% reduction from BAU F-gas plus scenario: From 147 million tonnes in 2010 to 57 million tonnes or 60% reduction from BAU |
| USEPA and ADEME (USEPA and ADEME 2004) | 2002-2015 | Scenario 1/BAU 2015 - Recovery efficiency does not increase; - Impact of regulations related to refrigerant phase-outs are considered | In BAU, the global level of equivalent CO2 emissions remains more or less constant over the period 2002-2015 and is at the same level of 1.5 billion tonnes of CO2 in 2002 and 2015. |
|  |  | Scenario 2 2015 - Leak tightness is improved - Recovery efficiency is improved at servicing and end of life; recovery is applied for sectors where it was not done before - Technologies for reduction of refrigerant charge are introduced - Lower GWP refrigerants are preferred whenever possible | Increase in refrigerant emissions by 32%. Sectoral emissions reductions- domestic 31%. Commercial 24%. 26% decline in overall CO2eq emissions. |
|  |  | Scenario 3 2015 - Partial phase-out of HFCs - Stronger efforts on improving leak tightness, recovery efficiency, charge reduction and low GWP selection as assumed for scenario - Reduction in fugitive emissions through better servicing and use of control devices - Refrigerant recovery for all sectors - Operators assumed to be well qualified and equipped with adapted tools - Technological choices are based on selecting lowest possible refrigerant charges and low GWP refrigerants | Decline in refrigerant emissions by 7.3%. Sectoral emissions reductions- domestic refrigeration 52%. Commercial refrigeration 41%. In absolute terms, 53% decline in CO2 eq emissions. |
| Zhao, Zeng, & Yuan, (2015) |  | Studied the reduction in carbon footprint of refrigerants used in China’s air conditioning sector based on improvements in the recovery rate/efficiency. Assumed carbon footprint reductions for a 1kW cooling capacity AC using either low, medium or high-GWP refrigerants: 5% scenario- BAU scenario 15% scenario- short-term recovery goals of China 30% scenario- current refrigerant recovery levels in Japan 50% scenario- world best scenario | Under 15%, 30% and 50% recovery scenarios, the carbon footprints of high GWP (R-22/R410A) refrigerants decreased by approximately 5%, 12% and 20% respectively. However, for middle (R-32) or low GWP refrigerants (R-161 and R-290), there was little improvement in their carbon footprint due to improved recovery rates. Additionally, the indirect emissions from the disposal process for these refrigerants increased from zero to a larger value. |

### Trends to Accelerate Adoption

Considering that refrigerant banks are increasing and would increasingly contribute to a larger share of emissions, there is an urgent need for international action and attention for refrigerant management of these banks. Additionally expanding and supplementing the Montreal Protocol’s earlier investments in infrastructure, training and governance institutions is necessary to recover and destroy these ‘’reachable banks’’ in the near term which would reduce the costs of destroying the banks emitted after 2015 (EIA 2009). The adoption of refrigerant recovery must be considered by countries as resource recovery. The penetration of refrigerant recovery technologies in developing country markets needs to be supported through strong regulations and laws which would provide a signal to industry to adopt more responsible approaches towards managing refrigerant use. In a study (Zhao, Zeng, and Yuan 2015) the impact of varying recovery rates on GHG emissions reduction was assessed. Refrigerant recovery rates of 5% (BAU), 15% (recovery goals of China), 30% (current recovery level in Japan) and a best-case scenario of 50% recovery globally was estimated for low, medium and high-GWP refrigerants used in room ACs.

Overcoming financial barriers

Increasing the availability and improving the accessibility of funds is one way to encourage refrigerant recovery in article 5 countries. Some sources of finance are described below:

Multilateral Fund (MLF)

To encourage refrigerant destruction, A5 parties require financial assistance for refrigerant recovery, transport, storage and destruction as well as technology transfer and cooperation (World Bank 2010). The Multilateral Fund (MLF) established after the Montreal Protocol to meet technical and financial needs of developing countries provides assistance under three core areas as given in Table 6.

Table 6: Core areas of financial and technical assistance by MLF

| Production | Consumption | Consumption and Technical Assistance |
| --- | --- | --- |
| Cost of conversion of existing production facilities | Cost of conversion of existing production facilities | Cost of premature modification or replacement of user equipment |
| Costs arising from premature retirement or enforced idleness | Costs arising from premature retirement or enforced idleness | Cost of collection, management, recycling if cost-effective, destruction of ODS |
| Cost of establishing new production facilities for substitutes | Cost of establishing new production facilities for substitutes | Cost of providing technical assistance |
| Net operational cost, including the cost of raw materials | Net operational cost, including the cost of raw materials |  |
| Cost of import of substitutes |  |  |

The fund has contributions from UNEP, UNIDO, World Bank and UNDP. The fund’s prime objective is meeting incremental costs incurred by A5 countries to comply with ODS phase out obligations. With respect to ODS destruction, MLF supports transport, storage and destruction but not collection of refrigerants for projects at a maximum level of $ 13.2/kg ODS destroyed. There are guidelines for funding of disposal projects under MLF. It is recommended that MLF expands its funding scope to include financing recovery/collection projects as well.

Economic incentives: There is a need to provide economic incentives for the refrigerant that is recovered and returned. According to IPCC-TEAP a survey conducted by RRA in 2003 showed that doubling of the financial incentive to service companies for returned refrigerant resulted in an increase of more than 50% of refrigerant returned (Metz et al. 2005). At the most basic level, end-users should not have to pay for refrigerant recovery and consumers could be offered a rebate on returned refrigerants (ICF International 2008).

Overcoming infrastructure, geographical barriers:

According to TEAP the end-of-life management of refrigerants in commercial refrigeration sector can be done by targeting national supermarket chains owned by large corporations (TEAP 2009). Regulations mandating safe disposal of used or contaminated refrigerants can be enforced on these corporations. To overcome the geographical barriers to refrigerant recovery and destruction imposed by the wide distribution of refrigerant banks and destruction facilities, it is important that shipment distances for products and equipment are minimized. This can be done by having multiple collection points. At these collection points the ODS can be recovered in concentrated form and transported to the nearest destruction facility. At the same time, tracking of materials collected is important especially where shipping distances are long and or transport is intermodal.

### Barriers to Adoption

In a review of the status of ODS recovery in EU countries, it was observed that even within the EU, there are variations in the degree to which refrigerant recovery, recycling, reclamation and destruction of F-gases is practiced (ICF International 2008). In a similar study of article 5 and non-article 5 countries, ICF International concluded that there are common barriers across developing countries. These are primarily technical, infrastructure, economic, regulatory and informational in nature. Below is a description of key barriers to refrigerant recovery:

Regulatory barriers: In most article 5 countries, regulations on refrigerant recovery are non-existent. Regulations prohibiting venting of refrigerants are also absent. If regulation is the key intervention in managing refrigerants end-of-life, strong enforcement is a precondition for success. This is particularly true where the economics of recovery become a disincentive for regulatory compliance (Devotta, Asthana, and Joshi 2004). Global experience in recovery and recycling (R&R) shows that R&R cannot be steered by price incentives alone and needs to be complimented by strong regulations.

Financial barriers: The construction and operating costs of refrigerant recovery, recycling, reclamation and destruction facilities is still quite high. For smaller appliances, such as in domestic uses, the costs of recovery greatly outweigh the benefits from refrigerant recovery. It is observed that all countries face funding constraints for destruction of refrigerants (World Bank 2010). TEAP (2009) has estimated that the costs to manage ODS from ‘’low effort’’ and medium effort banks of refrigerants and foam in all types of refrigeration equipment. According to the study, the potential funding requirement is likely to reach USD 850 million per year by 2015 in developing countries for “low effort” alone. The cost-effectiveness of managing refrigerants (CFCs and HCFCs) reaching waste stream is likely to decline over time with decline of proportions of these gases in the waste streams over time.

Informational barriers: Service technicians need to be equipped and trained and have to be informed of the safety risks associated with the handling refrigerant gases. There can also be informational barriers for large consumers or end users on the safe disposal of equipment at end of life, including the location of nearest reclamation/destruction facilities (ICF International 2008). Additionally, due to absence of national certification procedures for personnel, availability of trained man power is also a barrier.

Technology, infrastructure and geographical barriers: Recovery equipment and logistics (e.g. identifying cylinders or tanks for storage and transportation of ODS containing equipment or cylinders once material is recovered) are the building blocks to refrigerant recovery. Exporting appliances at the end of life for managing for recovery of refrigerants to other countries can lead to delays, leakage-emissions and other administrative complications with international conventions like Basel Convention. In such a scenario, domestic capacities need to be developed. However in most developing countries, facilities for destruction are located in remote locations hundreds of kilometres apart (ICF International 2008). This effectively either increases the costs of recovery or encourages venting. The construction and operating costs of facilities is also expensive. To some extent this barrier can be overcome by the construction of smaller, cheaper plants or mobile units that can destroy ODS at rates of 5kg/hour, reducing capital costs of facilities (Metz et al. 2005). The recovery/reclamation and destruction facilities must be well distributed geographically to prevent long-distance travel for equipment. There should be national facilities with the capacity to handle large amounts of refrigerants. However, most article 5 countries do not have their own high-capacity destruction facilities. Cement kilns in these countries can function as facilities to handle and destroy ODS (refrigerants) but any such plan must be supported by a sustainable business model (ICF International 2008).

End-use barriers: It may not be feasible to recover refrigerants from all sectors. For domestic refrigerators that contain small amounts of refrigerants (range of 50-350g per unit), recovery of refrigerants is still considered to be economically less viable. There are also refrigerant losses associated with the transport of the units to a recovery facility. Faced with scarcity of resources and time, countries should prioritize end-uses for refrigerant recovery. In Japan, significant quantities of refrigerants have been recovered from commercial sector.

Efficiency barriers: As to the efficiency of the refrigerants, it is difficult to assign energy efficiency to a refrigerant, because energy efficiency of refrigeration systems is an additional variable to the refrigerant choice also related to system configuration and component efficiencies. One approach when assessing the energy efficiency related to a refrigerant is to start with a specific refrigerant and use a system architecture suitable for this refrigerant while comparing to a reference system for the refrigerant to be replaced. Other approaches screen alternative refrigerants suitable for given system architecture. The common methods can be divided into theoretical and semi-theoretical cycle simulations, detailed equipment simulation models, and laboratory tests of the equipment. In practice, the achievable energy efficiency is limited by the cost of the system, as the success in the market depends on a cost-performance trade-off (UNEP 2016).

Cost Barriers: While some low-GWP refrigerants may cost more than the current baseline refrigerants, the cost of refrigerants is only ~1% of lifecycle costs for an AC. Costs can also be reduced through advanced manufacturing techniques and efficiency improvements, particularly if they are supported by policies that encourage technological development through deployment of superefficient ACs at scale. The relative contribution of equipment, refrigerant, installation and operating costs to customer lifecycle A/C costs for two systems with two different commonly used refrigerants is presented in Figure 3.

Figure 13: Residential life cycle A/C cost breakdown example (Goetzler et al. 2016)

In relation to the barriers to adoption of LGWP alternatives, Velders et al. indicate that:

Some alternatives to high-GWP HFCs have higher flammability than most HFCs, ammonia and some HFOs have lower flammability, and CO2 is non-flammable. Safety ratings also indicate higher toxicity for ammonia than for most HFCs and lower toxicity for hydrocarbons, HFC-1234yf, and CO2 (ASHRAE, 2016) .

Certain HFCs degrade in the environment to produce trifluoroacetic acid (TFA), a persistent toxic chemical. Atmospheric degradation of HFC-1234yf, a replacement compound for some long-lived HCFCs and HFCs, produces TFA. Potential impacts beyond a few decades of this TFA source could require future evaluation due to the environmental persistence of TFA and uncertainty in future emissions of HFC-1234yf and other HFCs that produce TFA upon degradation (WMO 2018).

More supporting work in the area of standards development and code adoption is needed for use of some of the lowest GWP options and fastest growing sectors, such as certain air conditioning sectors.

## Advantages and disadvantages of Refrigerant Management

Refrigerant replacement with low GWP refrigerants

The significant advantages of natural refrigerants such as ammonia and CO2 are their low GWP that suggests leaks will not damage the atmosphere or ozone, and the thermal properties of the refrigerants indicate that machine room equipment can be smaller and subsequently more cost-effective. Specifically, in the case of ammonia, the efficiency of the system is generally higher than with synthetic refrigerants offering further electricity savings.

Despite the benefits, there are two key disadvantages to using ammonia as a refrigerant. Ammonia is not compatible with copper, so it cannot be used in any system with copper pipes. Also, ammonia is poisonous in high concentrations. Two factors, however, mitigate this risk of ammonia usage. ammonia’s distinctive smell is detectable at concentrations well below those considered to be dangerous, and ammonia is lighter than air, so if in case of any leakage, it will rise and dissipate in the atmosphere (Shecco, 2017).

Carbon dioxide (CO2) is safer than ammonia. However, the high operating pressures required in the system can lead to leaks occurring more frequently. The high operating pressures can harm the system as changes in operating conditions can cause unwanted phase changes at different points in the system resulting in damage to system components. Another disadvantage of CO2 refrigeration higher system cost and less general familiarity with utilized components by many operators and service repairs providers.

As a refrigerant, ammonia has four significant advantages over CFCs and HCFCs. An ammonia-based refrigeration system costs 10-20% less to build than one that uses CFCs because narrower-diameter piping can be used. Also, ammonia is  3 to 10% more efficient than CFCs as a refrigerant, hence an ammonia-based system requires less electricity, resulting in lower operating costs. Plus, ammonia is substantially less expensive than CFCs or HCFCs and is safe for the environment, with an ODP and a GWP rating of zero.

Recovery and destruction of refrigerants at EOL

Refrigerant recovery followed by recycling, reclaiming and destruction, has the following benefits:

- Saves money: In cases where the recovered refrigerant is recycled and put back into the same system, it prevents costs incurred due to replacement of the refrigeration system

- Extended life of system: Reusing the same refrigerant from which impurities have been removed improves system efficiency, significantly increasing the life of the system

Since recycling does not improve impurities, reclamation is done to process the refrigerant back to original specifications. Reclaimed refrigerant can then be used in other systems or applications after being resold. Figure 14 illustrates the linkages between recovery, recycling and reclamation. When the refrigerant can neither be recycled nor be reclaimed, it is destroyed preventing its atmospheric release.

Figure 14: Linkages between recovery, recycling/reusing, reclamation and destruction (Source: Linde AG, n.d.)

Good management practices

There is a need to promote sound refrigerant management and conservation techniques that minimize emissions during these times. The section below describes approaches to reduce emissions during the working life of the equipment.

The report of the Refrigeration, Air-conditioning and Heat Pumps Technical Committee (RTOC, 2015) recommends the following approaches for refrigerant conservation:

- Reducing emissions through charge minimization/optimization- Both undercharging and overcharging of refrigeration equipment leads to GHG emissions. Undercharging impacts the efficiency of the unit leading to more indirect emissions from increased energy consumption and overcharging can lead to increased emissions throughout the active life of the unit. It is advisable that the quantity of refrigerants filled or ‘’charged’’ in a refrigeration equipment is reduced as it leads to lower emissions along the whole chain of refrigerant production, transport and service and at end of life.  It also results in a lower production cost and fewer risks in case of leakages of flammable or toxic refrigerants. Typically, the charge quantity is only specified for small equipment where performance is affected by charge quantity. However, in other equipment, the quantity of refrigerant being charged is rarely specified or pre-calculated. This can lead to overcharging, especially in systems with receivers as the amount of refrigerant contained in these receivers is not always known. The refrigeration industry is gradually recognizing the importance of charge minimization. For example, supermarket refrigeration systems designed with secondary loop and cascade systems use much smaller amounts of refrigerants.

- Reducing emissions through leak tightness: Preventing leakages is an important step towards refrigerant conservation. Leak detections must be done by manufacturers, at the end of assembly in field by contractors and during regular scheduled maintenance of systems by service technicians. Industry has also responded to improving leak tightness through better design.

- Reducing emissions during installation, servicing and maintenance: Using proper installation techniques during the time of installation of refrigerators can also contribute to refrigerant conservation. Servicing and maintenance activities must be conducted by trained service personnel. Training of technicians handling refrigerants is essential. They should be trained on handling and conservation of refrigerants, optimizing operation of equipment, control and reduction of energy consumption. Further they should be also aware of environmental and safety hazards of refrigerant handling.

### Additional Benefits and Burdens

Refrigerant recovery is resource recovery and avoiding refrigerant emission is resource conservation. Proper refrigerant recovery prevents the use of virgin refrigerants after recycling and or reclamation and prevents waste refrigerants from being vented out. From a life cycle perspective, this has significant GHG reduction benefits. It is to be noted that the environmental benefits from refrigerant destruction are likely to change over time as the composition of recovered refrigerants changes from CFCs to HCFCs. Apart from the environmental and economic benefits, there are social co-benefits through the creation of jobs and opportunities in the recovery sector.

# Methodology

## Introduction

Project Drawdown’s models are developed in Microsoft Excel using standard templates that allow easier integration since integration is critical to the bottom-up approach used. The template used for this solution was the Reduction and Replacement Solutions (RRS) which accounts for reductions in energy consumption and emissions generation for a solution relative to a conventional technology or approach. These technologies are assumed to compete in markets to supply the final functional demand which is exogenous to the model, but may be shared across several solution models. The adoption and markets are therefore defined in terms of functional units, and for investment costing, adoptions are also converted to implementation units. The adoptions of both conventional and solution were projected for each of several scenarios from 2015 to 2060 (from a base year of 2014) and the comparison of these scenarios (for the 2020-2050 segment) is what constituted the results.

The refrigerant management solution takes into account all CFC, HCFC and HFC gases but does not include PFC, SF6 and other F gases. The solution is restricted to refrigeration, heat pumps and AC applications and does not cover other uses of F gases such as in solvents, firefighting systems, industrial uses, foam blowing agents, MDI etc.

The adopted model for the refrigerant management solution is shown in Figure 7. Annual production of the refrigerants contributes to increasing refrigerant banks which are estimated to be about 5-10 times the annual production of refrigerants. The refrigerant bank is growing in size as the production rate of equipment is higher than the retirement rate of equipment leading to increased banking of refrigerants. This is due to increasing use of AC and refrigeration systems in emerging economies which is directly proportional to economic prosperity. Refrigerants are emitted into the environment during the production process, from refrigerant banks due to leakages and during end of life disposal of the appliances (shown as EOL banks in Figure 7). Refrigerant banks of HFC decline as the forecast for adoption of low GWP HFC replacements enter the market and the rate of production of HFC begins to decline relative the retirement of appliance using HFC refrigerants at EOL.

As shown in Figure 7, refrigerant management can be undertaken in five ways:

- Lowering demand/use of appliances and thereby production.

- Replacing refrigerants with low GWP HFC/new cooling agents/non-HFC substances.

- Increasing efficiency of refrigeration in appliances thereby lowering the use of refrigerants.

- Controlling leakages of refrigerants from existing appliances by good management practices.

- Ensuring recovery, reclaiming/recycling and destruction of refrigerants at EOL.

This report and solution is restricted to sl. (ii.), (iv) and (v) only.

## Data Sources

In order to develop the potential market for adoption of Refrigerant Management several calculations and transforms were required to take global data from a number of sources to develop a forecast for the growing size of refrigerant banks across multiple classes of refrigerants and by subsectors and appliance use cases.  These key data sources include, Velders et al., 2009, Gschrey and Schwarz, 2009, Purohit and Isaksson, 2016, Velders et al., 2014, Velders et al., 2015 and Clodic et al., 2010, Scientific Assessment of Ozone Depletion 2018 (SAD).  In addition to these sources, two other received directly from Velders were added: Post-Kigali projections dated Velders et al. (2017) and the background data for Figure 2-2 from Chapter 2 of the SAD, 2018.

Velders et al. (2015) data includes new scenarios of HFC growth that are specific to use sectors and geographic regions The inclusion of more detailed and comprehensive data for different use sectors and regions increases confidence in projected HFC emissions in coming decades and in estimates of the effectiveness of existing and proposed control measures. This new scenarios, just as in Velders 2009, can be qualified as business-as usual (BAU) scenarios because they assume that current uses (substances and technologies) of HFCs for specific sectors continue unabated and that developing countries follow the same transitions from HCFCs to HFCs and not in-kind (NIK) alternatives as has occurred in developed countries (Velders et al. 2015).

In the IPCC Special Report on Emissions Scenarios (SRES) (IPCC,2000), used in the previous analysis (Velders et al., 2009), GDP and population data are available only for four regions, while the SSP scenarios are available for 32 countries/regions, which are aggregated to 11 countries/regions for this study. IPCC/SRES has four scenarios for GDP and population, while the SSPs has five scenarios, each quantified by three groups. In this analysis, we use the five datasets (SSP1 to 5) quantified by the OECD as illustrative SSPs as recommended on the International Institute for Applied System Analysis (IIASA) website (O'Neill et al., 2012). For the HFC scenarios developed by Velders, SSP5 is the upper range scenario and SSP3 is the lower range scenario. Technological developments are not driven by environmental concerns in these SSPs and hence are not taken into account in the formulation of the HFC baseline scenarios.

Figure 15: HFC emissions (GtCO2-eq yr−1) from 2000-2050 (Data source: Velders et al., 2015)

Figure 15 shows one of the estimates of the quantity of different refrigerants emitted from 2000 to 2050 (Velders et al., 2015). Refrigerant emissions from CFC are estimated to fall after 2010 as their production has been phased out. However, the refrigerants are banked in some appliances and these accounts for the emissions from leakages and EOL disposal. The contribution from developing countries is split into that from historical HFC production (1990–2012) and its projection (starting in 2013) and that from replacement of HCFCs (starting in 2013). Emissions of HFCs, continue to grow till 2050, where in developed countries, emissions are 0.8–1.0 GtCO2-eq yr−1.

Refrigerants use is sub-sector specific and the HFC emissions and refrigerant banks from various sub-sectors estimated for 2050 is shown in Table 7. Table 7 also highlights that emissions as a percentage of banks is high for commercial, refrigerated transport and mobile ACs. It was estimated that 1380 ktons of refrigerants are likely to be emitted in 2050 which is 16.5% of the total banks existing in 2050. It should be highlighted that these estimates are only for HFCs.

Table 7: Forecast of emissions and refrigerant banks in 2050 (HFC only) (Data source: Gschrey and Schwarz, 2009)

|  | Emissions (Ktons) | Banks (Ktons) | Share of emissions as a % of banks | Emissions (MT CO2 eq.) |
| --- | --- | --- | --- | --- |
| Commercial Ref | 511 | 2402 | 21.27% | 1638 |
| Domestic Ref | 10 | 166 | 6.02% | 14 |
| Refrigerated transport (excluding shipping) | 16 | 58 | 27.59% | 31 |
| Industry (cold storage) Ref | 59 | 457 | 12.91% | 145 |
| Stationary AC (air to air) | 415 | 3610 | 11.50% | 854 |
| Mobile AC | 369 | 1667 | 22.14% | 528 |
| Total | 1380 | 8360 | 16.51% | 3210 |

Figure 16 shows the estimated growth of emissions of HFC (only) from various sub-sectors in Gton CO2 eq. per year from one of the literature sources.  The large and growing share of commercial refrigeration and stationary AC systems to the refrigerant emissions is to be noted.

Figure 16: Estimated emissions of HFC (only) from various sub-sectors (Data source: Purohit and Isaksson, 2016)

Figure 17: Estimated refrigerant banks from 2000-2050 (Source: Velders et al., 2017)

Figure 17 shows the estimated refrigerant banks from 2000 to 2050 from one of the data sources. It is to be noted that the scale of the chart in Figure 17 is six times that in Figure 15. Current forecasts for quantities of refrigerants emitted annually, are in the range of one fifth to one sixth of the existing refrigerant banks.

Following units are adopted for the solution:

Functional unit1: Kilo tons of refrigerant (HFC refrigerant replaced by low GWP refrigerant)

Implementation unit1: Kilo tons of refrigerant

Functional unit2: Kilo tons of refrigerant (which is vented but is recoverable)

Implementation unit2: Kilo tons of refrigerant (venting of which is avoided by adopting refrigerant management practices like leakage prevention during use and refrigerant which is destroyed at EOL)

## Total Addressable Market

All refrigerants which are produced in a given year are potential sources of GHG emissions. However, these end up in refrigerant banks in appliances and become sources of leakages during the lifetime of the appliances and at end of life disposal from venting of refrigerants. Estimates of GHG emissions from refrigerants are available in literature (these include both leakages from ‘Refrigerant banks’ and ‘EOL emissions’) and estimates from CFC, HCFC and HFCs are considered separately in modelling this solution. The three families of refrigerants are considered separately as they have different GWP and therefore different potency to impact climate.

The total GHG emissions are calculated from Eq (1)

Total GHG emissions = Quantity of refrigerantF * Average GWP of refrigerantF		(1)

Where F is the type of refrigerant: CFC, HCFC, HFC

As noted from Fig. 14, HFCs would form the major component of refrigerants which are emitted in 2045. It is assumed that all emissions during the considered period are from a standard refrigerant which has an average GWP factor of 2326. This factor is obtained from estimates in table 6 using Eq(2).

Average GWP factor = Emissions (MT CO2 eq)/ Emissions (Ktons) = 2326.087		(2)

For the HFC replacement model Velders (2015) provides refrigerant emissions (measured in kton refrigerant – HFCs).  Velders provides and upper scenario and a lower scenario.  A mean scenario is also calculated.  These forecasts are pre-Kigali and show a steadily increasing HFC emissions profile through 2050.  These scenarios become the basis for the TAM for the replacement model.

Velders (2017) provides a revised forecast for emissions of ODS and HFCs – post Kigali accord.  This would account for an adoption of low GWP HFC replacements.  The two scenarios (upper and lower) becomes the revised (integrated) TAM for the good management practices model.  These show a decreasing emissions (kton refrigerant) due to the decrease in HFC production and banks through 2050.

## Adoption Scenarios

Two different types of adoption scenarios were developed: a Reference (REF) Case which was considered the baseline, where not much changes in the world, and a set of Project Drawdown Scenarios (PDS) with varying levels of ambitious adoption of the solution. Published results show the comparison of one PDS to the REF, and therefore focus on the change to the world relative to a baseline.

### Reference Case / Current Adoption

In the Reference Case (REF) scenario for the HFC replacement model, Velders (2017) provides an adoption of low GWP HFC replacements forecast that includes a value for 2014.  That value of emission from 2014 is divided by the TAM for 2014 to derive a percentage adoption – Current Adoption.  This percentage is applied to the TAM for each year in the period of study and becomes the REF scenario.

For the management practices model, to develop the REF scenario, it is assumed that the solution consists of only the one aspect of recovery of gases and its destruction from appliances at EOL. Leakage detection and repair may be practiced in some sectors, but this is considered as the baseline. Hence in the REF case it is assumed that leakages from ‘Refrigerant banks’ continue unabated.

Table 8: Estimated HFC refrigerant banks from 2010 to 2050 (Gton CO2 eq.) (Data source: Velders et al., 2015)

| Year | Commercial and industrial | Domestic Ref | Stationary AC | Mobile AC | HFC Total |
| --- | --- | --- | --- | --- | --- |
| 2010 | 2 | 0 | 1 | 0.5 | 3.5 |
| 2020 | 6 | 0.5 | 4 | 1 | 11.5 |
| 2030 | 12 | 0.5 | 10 | 1 | 23.5 |
| 2040 | 18 | 1 | 18 | 1.5 | 38.5 |
| 2050 | 22.14 | 1.08 | 22.14 | 2.16 | 54 |

Table 8 shows the estimated HFC refrigerant banks from 2010 to 2050 (Upper range) in Gton CO2 eq. This data is used to estimate the quantity of refrigerant banks (ktons) from 2010 to 2050 assuming that the standard refrigerant having a GWP of 2326 is used. The estimated adoption of the solution in the REF scenario from 2010 to 2050 (ktons) is thereafter calculated, assuming past history of recovery rates of refrigerants at EOL. These recovery rates are calculated for various sub-sectors from data reported for 2006 (Clodic et al., 2010).  These rates are kept constant and are applied to the estimated data banks in future years to calculate the adoption scenario in the REF case.

The estimated refrigerant banks (CFC, HCFC and HFCs), estimated emissions of refrigerant and recovery from various sub-sectors in 2006 are available for the year 2006 (Clodic et al., 2010).  Using this data, emissions and recovery of refrigerants as a % of total banks are calculated. The assumed recovery factors as a % of total banks for different sub-sectors of HFC, CFC and HCFC are shown in Table 9.

Table 9: Assumed recovery factors at EOL for various sub-sectors (as % of total banks)

|  | HFC |  |  |  | CFC | HCFC |
| --- | --- | --- | --- | --- | --- | --- |
| Sub-sector | Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | All | All |
| Recovery factor | 1.44% | 0.07% | 1.22% | 0.18% | 1.54% | 1.56% |

The estimated banks of CFC and HCFCs from 2010 to 2050 in Gton CO2 eq. is available in Velders et al., 2014. Similar to the case for HCFCs, this data is used to estimate the quantity of refrigerant banks (ktons) from 2010 to 2050 assuming that the average GWP of CFC is 9292 and the average GWP of HCFC is 1502. These average GWP for different refrigerants are calculated based on the estimates of total emissions from different refrigerants (Gschrey and Schwarz, 2009). The estimated adoption of the solution in the REF scenario from 2010 to 2050 (ktons) for CFC and HCFCs is calculated, assuming past history of recovery rates shown in Table 9. ICF International (2008) estimates that approximately 30% of the recovered refrigerant is reclaimed or destroyed in the EU, while the remining 70% is recycled. Further, according to the World Bank (2010), destruction rates are higher in non A5 countries but at the global level, destruction rates could vary from 1% to 50%. Using this data, the destruction rate of 25% is fixed in the REF scenario and the estimated destruction of CFC, HCFC and HFC from 2010 to 2050 is calculated in the model.

The respective quantity of refrigerants which are estimated to be destroyed in the REF case are multiplied by the GWP of the refrigerants to calculate the estimated emissions (ktons CO2 eq.) which are avoided. This quantity is then divided by 2326 to calculate the estimated quantity of standard refrigerant (ktons) which is destroyed.

### Project Drawdown Scenarios

Three Project Drawdown scenarios (PDS) were developed for each solution, to compare the impact of an increased adoption of the solution to a reference case scenario.

Each PDS scenario is composed of three parts. The first part of the solution is the attribution to the adoption of low GWP refrigerants in replacement of HFC. By replacing HFC, the expected emissions from current rates of leakage are reduced. The second part of the solution in each scenario is calculated from increased adoption of recovery and destruction of refrigerants at end of life of an appliance. Higher recovery rates, increased recycling/reclaiming and higher destruction rates for these refrigerants can reduce EOL emissions. Table 10 shows the estimated banks of CFC, HCFC and HFC (ktons) which are retiring every year at EOL.

Table 10: Estimated refrigerant banks retiring at EOL (ktons)

|  | HFC |  |  |  | CFC | HCFC |
| --- | --- | --- | --- | --- | --- | --- |
| Year | Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | All | All |
| 2010 | 85.98 | 0 | 42.99 | 21.50 | 94.16 | 233.02 |
| 2020 | 257.95 | 14.33 | 171.96 | 42.99 | 47.08 | 291.27 |
| 2030 | 515.90 | 14.33 | 429.92 | 42.99 | 23.54 | 145.63 |
| 2040 | 773.86 | 28.66 | 773.86 | 64.49 | 18.83 | 116.51 |
| 2050 | 951.84 | 30.95 | 951.84 | 92.86 | 0 | 58.25 |

This is calculated from estimates of banks and retirement rate of appliances used in various sectors. To estimate the retirement rates from various sub-sectors we take the lifetime of the equipment into account. These are shown in Table 11 below. For CFC and HCFC, average of all sub-sectors is used.

Table 11: Retirement rate of banks (Data source: Purohit and Isaksson, 2016)

|  | HFC |  |  |  | CFC | HCFC |
| --- | --- | --- | --- | --- | --- | --- |
|  | Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | All | All |
| Lifetime of eqpt (Yrs) | 10.00 | 15.00 | 10.00 | 12.00 | NA | NA |
| % of eqpt retiring every year | 10% | 6.67% | 10% | 8.33% | 8.75% | 8.75% |

Table 12: Estimated EOL recovery factorsPDS case (%) (Data source: Purohit and Isaksson, 2016 for HFC; Average of all sub-sectors for CFC and HCFC)

|  | HFC |  |  |  | CFC | HCFC | HFC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | All | All | Average of all sub-sectors |
| 2010 | 14.45% | 1.03% | 12.16% | 2.13% | 17.61% | 17.84% | 7.44% |
| 2020 | 36.30% | 27.35% | 37.44% | 22.42% | 38.41% | 38.56% | 30.88% |
| 2030 | 58.15% | 53.68% | 62.72% | 42.71% | 59.20% | 59.28% | 54.31% |
| 2040 | 80.00% | 80.00% | 88.00% | 63.00% | 80.00% | 80.00% | 77.75% |
| 2050 | 80.00% | 80.00% | 88.00% | 63.00% | 80.00% | 80.00% | 77.75% |

Table 12 lists the estimated EOL recovery factors (%) for all PDS scenarios. It is assumed that the recovery factors in 2010 were those observed in the past (data for 2006 which was reported as ‘recovery as a % of total banks’ has been converted to ‘recovery as a % of retiring banks’ by using the ‘% of equipment retiring every year’ and is retained for 2010). It is further assumed that by 2040 maximum feasible recovery factors are reached (Purohit and Isaksson, 2016). Recovery factors for intermediate years (2020 and 2030) are derived by interpolation while that for 2050 is the same as that for 2040.  The estimated recovery of CFC, HCFC and HFCs at EOL from 2010 to 2050 (ktons) have then been calculated from refrigerant banks and recovery factors in Table 13.

Table 13: Estimated recovery of CFC, HCFC ad HFCs at EOL from 2010 to 2050 (ktons)

|  | HFC |  |  |  | CFC | HCFC | HFC |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | All | All | Average of all sub-sectors |
| 2010 | 12.42 | 0 | 5.23 | .38 | 16.58 | 41.57 | 7.93 |
| 2020 | 93.63 | 3.92 | 64.38 | 8.03 | 18.08 | 112.31 | 108.41 |
| 2030 | 299.98 | 7.69 | 269.64 | 15.30 | 13.94 | 86.33 | 439.27 |
| 2040 | 619.07 | 22.93 | 680.97 | 33.86 | 15.07 | 93.21 | 1116.26 |
| 2050 | 761.45 | 24.76 | 837.60 | 48.75 | 0 | 46.60 | 1530.60 |

The estimated destruction of different refrigerants (kton) has then been calculated from the estimated recovery of refrigerants using destruction factors shown in Table 14.

Table 14: Assumed destruction rates in recovered refrigerants (% of total recovered)

| Year | HFC | CFC | HCFC |
| --- | --- | --- | --- |
| 2010 | 25.00% | 25.00% | 25.00% |
| 2020 | 36.67% | 50.00% | 43.33% |
| 2030 | 48.33% | 75.00% | 61.67% |
| 2040 | 60.00% | 100.00% | 80.00% |
| 2050 | 60.00% | 100.00% | 80.00% |

25% destruction rate is taken as the starting point for year 2010 for all refrigerants. It is assumed that the destruction of recovered refrigerants increases to its maximum feasible value by 2040. CFCs have been phased out and hence these cannot be recycled. Hence it is assumed that all CFC which is recovered would be destroyed. HCFCs would also be phased out by 2030. However, a small portion of the recovered refrigerant may be used for servicing and other requirements. Hence an 80% destruction rate is assumed for the refrigerant. HFCs are likely to be used as refrigerants beyond 2040 and their recovery would continue to make business sense.  Hence a 60% destruction rate has been assumed. Recovery factors for intermediate years (2020 and 2030) are derived by interpolation while that for 2050 is the same as that for 2040.

The third part of the solution in each PDS scenario is contributed by adopting good practices. A large share of refrigerant leakages can be prevented by adopting this part of the solution. Table 15 shows the estimated emissions as leakage from banks (kton).

Table 15: Estimated emissions as leakage from banks (kton)

|  | CFC | HCFC | HFC |
| --- | --- | --- | --- |
| Year | All | All | Average of all sub-sectors |
| 2010 | 202.08 | 371.09 | 280.10 |
| 2020 | 101.04 | 463.87 | 760.08 |
| 2030 | 50.52 | 231.93 | 1387.85 |
| 2040 | 40.42 | 185.55 | 1435.16 |
| 2050 | 0.00 | 92.77 | 1043.95 |

This is calculated from estimated banks of CFCs, HCFCs and HFCs and using the estimated leakage factors (derived from observed data in 2006) shown in Table 16. This factor is a combination of leakages and emissions from disposal of equipment at EOL. In absence of any additional estimates from the literature, this % is taken as the as the leakages during the lifetime of the equipment.

Table 16: Assumed leakage factors for different sub-sectors (as % of total banks)

| HFC |  |  |  | CFC | HCFC | HFC |
| --- | --- | --- | --- | --- | --- | --- |
| Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | All | All | Average of all sub-sectors |
| 19.30% | 6.39% | 11.22% | 18.78% | 18.78% | 13.93% | 13.92% |

The estimated emissions of refrigerants (ktons) which can be avoided from good in-use management is calculated from refrigerant banks and the estimated removal efficiency factors which can be achieved.  The assumed removal efficiency factors are shown in Table 17.

Table 17: Estimated 'in use’ removal efficiency factors PDS case (%) (Data source: Purohit and Isaksson, 2016 for 2040 and beyond; others: linear interpolation)

|  | HFC |  |  |  | CFC | HCFC | HFC |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | All | All | Average of all sub-sectors |
| 2010 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| 2020 | 11.00% | 26.67% | 10.00% | 16.67% | 0.00% | 16.67% | 16.08% |
| 2030 | 22.00% | 53.33% | 20.00% | 33.33% | 0.00% | 33.33% | 32.17% |
| 2040 | 33.00% | 80.00% | 30.00% | 50.00% | 0.00% | 50.00% | 48.25% |
| 2050 | 33.00% | 80.00% | 30.00% | 50.00% | 0.00% | 50.00% | 48.25% |

It is assumed that the baseline is 2010 where no measures were adopted to control leakages during the working life of the equipment. This baseline stays consistent in the REF scenario. The maximum 'in use’ removal efficiency factors for various sub-sectors are derived from Purohit and Isaksson (2016) and it is assumed that the best-case scenario can be attained by 2040. Removal efficiency factors for intermediate years (2020 and 2030) are derived by interpolation while that for 2050 is the same as that for 2040.  For CFCs it is assumed that there would be no change in the removal efficiency factor as these have been phased out and it is economically not viable to undertake improvements in these appliances and systems.

#### Plausible Scenario

Results from the three parts of the solution are added to generate the estimated solution in the Plausible Scenario. The Plausible Scenario takes the lower range adoption of low GWP HFC replacements.

The quantity of refrigerants in the Plausible Scenario managed by better practices are multiplied by the respective GWP of the refrigerants to calculate the estimated emissions (ktons CO2 eq.) which are avoided. This quantity is then divided by 2326 to calculate the estimated quantity of standard refrigerant (ktons) which can be managed. For HFCs, a sum of the various sub-sectors is used rather than the average across sub-sectors and the high and low HFC bank estimates from Velders et al., 2014. The table of results calculated for ktons of normalized (standard) refrigerant is then interpolated to develop the preliminary Plausible Adoption scenario table of year over year values.  This table is then modified by adjusting the 2014 value to match the current adoption calculated for by using the baseline adoption of destruction of revered refrigerant of 29.61 ktons.  The remaining year over year results are re-interpolated and a sigmoid best fit curve line is chosen to develop the final scenario table of values

#### Drawdown Scenario

The Drawdown Scenario uses the same inputs and assumptions and methodology as the Plausible Scenario, except that the average adoption of low GWP HFC replacement data is used rather lower bound scenario.  An identical process of modifying the final table to correlate with the current adoption in 2014 is used to calculate the final values.

#### Optimum Scenario

The Optimum Scenario uses the same inputs and assumptions and methodology as the Plausible Scenario, except that the higher bound adoption of low GWP HFC replacement data is used rather lower bound or averagescenario.  An identical process of modifying the final table to correlate with the current adoption in 2014 is used to calculate the final values.

## Inputs

### Climate Inputs

Direct emissions from refrigerants which leak during the lifetime and at EOL contribute to global warming. These are dependent on the quantity of refrigerants which are emitted in the environment and their respective GWP. For the refrigerant management solution, we assume that all emissions are from a “standard refrigerant” which has a GWP of 2326. Hence a value of 2326 tons CO2 eq. per ton of refrigerant is assumed as the Direct Emissions per Conventional Functional Unit. In case of the solution, refrigerant emissions are avoided and refrigerants are destroyed. Hence direct emission from the solution are considered as 0. There are indirect emissions when the solution is implemented from collection of refrigerants. However, these are assumed to be 0 due to lack of data. Other variables such as annual fuel consumed and CH4 tons reduced do not apply for this solution.

Electricity is consumed when refrigerant destruction facilities are operated. This is quantified for the solution as energy (kWh/kg) consumed in destruction processes. Table 18 lists TEAP approved technologies for refrigerant destruction. Incineration remains the most widely used method for destroying refrigerants. Incineration in rotary kilns and cement kilns is widely used in the US (USEPA 2009) and developing countries respectively. However in the EU, Reactor cracking and Plasma Arc (to some extent) are used (ICF International 2008).

Table 18: TEAP approved destruction technologies and energy consumption

| Technology | Destruction efficiency (%) | Energy consumption (kWh/kg CFC) | Availability |
| --- | --- | --- | --- |
| Incineration |  |  |  |
| High performance | 99.99 | 1.71 | commercial |
| Liquid injection | 99.99 | 1.3 | commercial |
| Rotary Kiln | 99.99 | 1.54 | commercial |
| Gas/fume | 99.99 | 1.3 | commercial |
| ICFB | 99.99 | 1.3 | commercial |
| Cement Kiln | 99.99 | 1.86 | demo |
| Reactor Cracking | 99.99 | 1.55 | commercial |
| Plasma |  |  |  |
| IC RF | 99.99 | 3.7 | Demo |
| Argon | 99.99 | 2.3 | commercial |
| AC | 99.99 | 2.1 | condition |
| Other |  |  |  |
| Solvated electron | 99.99 | 10 | demo |
| UV Photolysis | 99.99 | 10 | commercial |
| Gas phase chemical reduction | 99.99 | 1.38 | condition |
| Catalytic dehalogenation | 99.99 | 0.79 | demo |
| Liquid phase chemical conversion | 99.99 | 3 | commercial |
| Vitrification | 99.99 | 4.88 | condition |

Table 19: Climate Inputs

|  | Units | Project Drawdown Data Set Range | Model Input | Data Points (#) | Sources (#) |
| --- | --- | --- | --- | --- | --- |
| Total Energy Used per functional unit - SOLUTION | kWh/kg CFC | 1.3 - 10 | 2.7 | 9 | 1 |

Note: Project Drawdown data set range is defined by the low and high boundaries which are respectively 1 standard deviation below and above the mean of the collected data points.

### Financial Inputs

Refrigerant management is an uncommon solution in the sense that it has no cost savings and only net operating costs. For the solution, no direct upfront costs are applicable for control of leakages from appliances (Purohit and Isaksson, 2016). However, there are operational and maintenance costs per unit of refrigerant for adopting these measures. For the destruction of refrigerants, it is assumed that requisite facilities are already available. This assumption may be erroneous as there are few destruction facilities and upfront investment is required for construction of kilns across geographical locations especially in developing countries. However, considering the complications in estimating the required size and optimal siting of such facilities, it was decided to neglect these upfront costs compared to the operational costs of collection, recovery and destruction of refrigerants. Hence, only costs of collection, recovery and destruction are used as operational costs per ton of refrigerant. The remaining parameters such as learning rate and other factors are set to default.

There are different costs for each of the two parts of the solution. Adopting good practices is significantly cheaper than recovery and destruction of refrigerant. In order to derive a single variable for operating cost per functional unit of the solution, a set of custom calculations were developed.  Firstly, an average cost per Million Euro/yr per kiloton Refrigerant for good management practices was calculated by multiplying the annual cost estimates showing in Table 20 by the forecasted amount refrigerant (kton) leakage avoided at each decade from 2010-2050.  The cost is assumed to be the same per unit over the period.  An average is developed and then converted to USD to arrive at $2.8 Million USD/yr per kton refrigerant managed through good practices.

Table 20: Million Euro/yr per kiloton Refrigerant by Sub-Sector Data Source: (Purohit &Isaksson,2016)

| Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | CFC | HCFC |
| --- | --- | --- | --- | --- | --- |
| 2.1 | 12.5 | 1.4 | 0.6 | 3.5 | 3.5 |

The total costs for end of life recovery, removal and destruction are calculated in a similar way. Table 21  shows various estimates (low and high) for RRD from different sub-sectors.  Multiplying these values by the estimated destruction of refrigerant in the PDS scenarios and then taking an average over the entire period arrives at an average value of $45.41 USD/yr per kton refrigerant removed, recovered and destroyed. which is used for further calculations.

Table 21: Costs for removal and recovery in 2009 (Data Source: TEAP, 2009)

|  | Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | CFC | HCFC |
| --- | --- | --- | --- | --- | --- | --- |
| Lower Range | 46.5 | 41 | 22 | 11 | 30.13 | 30.13 |
| Higher Range | 90 | 73.5 | 30 | 12.5 | 51.5 | 51.5 |

The average operating cost per kton refrigerant was derived by taking the average for each average annual cost of refrigerant managed through good practices and the average annual cost of refrigerant removed, recovered and destroyed and multiplying them by the corresponding forecast of ktons refrigerant from the PDS scenarios and then taking an overall average from the entire period.  This figure is calculated to be $22.6 USD/yr per kton refrigerant managed.

The cost for HFC replacements with low GWP refrigerants was not considered in this analysis at this time.  Due to the heterogenous nature of both the appliances and the potential replacement refrigerants combined with the early stages of adoption of HFC replacements leaves a gap in the literature for reliable global estimates of costs of adoption of new appliances.

## Assumptions

Six overarching assumptions have been made for Project Drawdown models to enable the development and integration of individual model solutions. These are that infrastructure required for solution is available and in-place, policies required are already in-place, no carbon price is modeled, all costs accrue at the level of agency modeled, improvements in technology are not modeled, and that first costs may change according to learning. Full details of core assumptions and methodology will be available at www.drawdown.org. Beyond these core assumptions, there are other important assumptions made for the modeling of this specific solution. These are detailed below.

- There are various types of refrigerants existing in banks and these leads to different types of refrigerant emissions over different years. For this solution, it is assumed that there is only one type of (standard) refrigerant which has a GWP of 2326.

- It is assumed in the PDS scenarios, the maximum feasible rates of recovery efficiency, destruction efficiency and removal efficiency are met uniformly across the world. This assumption may lead to some overestimation of the potential of the solution as the implementation of the solution may be non-homogeneous across the world (lower in developing countries).  A regional assessment which captures different rates for different countries over time may improve the assessment.

- The current adoption of the solution is based on a report which assessed the refrigerant banks, emission and destruction of different refrigerants in 2006. While there would have been improvements over the last decade, there are no reliable reports on a global basis. Hence the adoption of refrigerant management solution in 2014 may be lower than the actual adoption.

- Under the PDS scenario, the recovery and destruction rates have been assumed to grow linearly. This is an optimistic assumption, since while recovery rates have improved over time, the growth may not be linear. The improvement in recovery and destruction is driven by regulations and incentives. Assuming all policies, regulations and programs are in place for encouraging or mandating recovery and destruction, linear rates of increase can potentially be optimistic in the short run and discounted in the long run.

- Fixed retirement rate to calculate refrigerant banks in EOL stage for each year. The mean lifetime of the equipment varies between regions and is dependent on several factors. For example, the average lifetime of the equipment can increased by proper servicing and replacement of refrigerants. However, in the model, common rates of retirement for different appliances were used, globally. While country level data may exist for capturing this variable, it is difficult to account for separately and hence a common rate is used globally.

- The assumption of 100% destruction of recovered refrigerants for CFC, 80% for HCFC and 60% for HFCs by 2040 at the world level is dependent on the assumption that best practices are deployed in destruction under which no leakages occur during storage, no emissions during transfer of refrigerants into a pressurized container, no emissions during transfer of contents from pressurized to non-pressurized container and no emissions occur during transportation to destruction unit. These are considered achievable as according to USEPA, if best practices are employed, emissions can be as small as about 2% to 3%. These high rates of destruction though possible, will be dependent on regulatory provisions and creation of financial incentives for destruction, since currently also it competes with recycling and reclamation. The analysis has assumed 20% of HCFC and 40% of HFC may be used for recycling and reclamation from the recovered refrigerants. Under an optimistic scenario of total emissions reduction possible from refrigerant recovery, 100% could have been considered.

## Integration

The complete Project Drawdown integration documentation (will be available at www.drawdown.org) details how all solution models in each sector are integrated, and how sectors are integrated to form a complete system. Those general notes are excluded from this document but should be referenced for a complete understanding of the integration process. Only key elements of the integration process that are needed to understand how this solution fits into the entire system are described here.

The main point of integration with Refrigerant Management and the other solutions is the effect of a changing grid.  As the associated renewable grid technologies are deployed it will effect the emissions factors for the indirect emissions created with grid electricity use to power the refrigerant recovery and destruction factories.

## Limitations/Further Development

Lack of data for key inputs imposed severe limitations on the analysis. As discussed in the previous sections, there were no comprehensive datasets on refrigerant banks or quantities of refrigerants potentially available for recovery. Data on refrigerant banks collected from several sources was either not disaggregated into end-use type/application or not disaggregated into gases (CFCs, HCFCs, HFCs etc) or not available at regional level. Only data points for 2006 on refrigerant banks was found to be somewhat complete (even though it was not disaggregated into gases). Data on refrigerant banks and the % of these banks available for recovery is a critical input for this analysis.

Indirect emissions from powering refrigeration appliances is a significant component of grid emissions and is expected to increase in a warming world.  Adoption of low GWP HFC replacements with new appliances brings the potential for increases in energy efficiency.  Some have estimated that the potential impact on indirect emissions may be half as much or as great as direct emissions reduction.  At this time this model and report does not include a forecast for indirect emissions reduction.  As the literature includes additional research on global estimates of indirect emissions reductions, this data will be included as part of this solution.

Refrigerant banks were not segregated into unwanted, non-reusable, non-reclaimable quantities due to lack of data at global as well as regional level. This type of segregation will also impact results. .

Refrigerant banks estimated using 2015 data covered only refrigerants banked in equipment stock and therefore the analysis does not consider other categories of refrigerants that may be available for destruction. These include:

- Refrigerant recovered from equipment in the past with incomplete documentation

- Refrigerant recovered from equipment after 2006 with appropriate documentation

- Stockpiles of mixed or contaminated refrigerants that cannot be reused

The analysis has been done in a keeping in view time and data limitations. Several assumptions have been used to model emissions reductions possible from refrigerant recovery and destruction at the global level and regional level. The analysis can benefit from improved or more data on key inputs like refrigerant banks, retirement rates, more informed assumptions on recovery and destruction rates. The main areas of model improvement include:

- Cross-checking or validating the overall bank estimates against any other available data (most at country level) or from national inventory reports.

- Review or validate the retirement rates assumed in this model. Find out if there is data from other countries or regions which can provide a better estimate of this retirement rate

- Modeling pathways of recycling, reclamation and destruction- this would be a more realistic assessment of emissions avoided from refrigerant recovery at the end of life of equipment

- Reviewing the estimates of emissions reductions based on possible phase out dates.

- It would also be interesting to see the impact of changing retirement rates across different countries on the total emissions reduced from refrigerant recovery and destruction.

- The outputs from this analysis can be improved with regional and updated data.

- Availability of additional data on costs can also affect the financial implications of implementing this solution.

# Results

## Adoption

Below are shown the world adoptions of the solution in some key years of analysis in functional units and percent for the three Project Drawdown scenarios.

Table 22: World Adoption of the Solution

| Solution | Units | Base Year (2014) | World Adoption by 2050 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Plausible | Drawdown | Optimum |
| Refrigerant Management | kton HFC replaced | 2.79 | 1453.4853 | 1778.4099 | 2103.3345 |
|  | (% market) | 0.7% | 67% | 82% | 97% |
|  | Kilo Tonnes (Refrigerant destroyed and avoided) | 29.61 | 742.02 | 742.02 | 742.02 |
|  | (% market) | 3.16% | 100% | 100% | 100% |

Figure 18: World Annual Adoption 2015-2060 HFC Replacement

## Climate Impacts

Below are the emissions results of the analysis for each scenario which include total emissions reduction, atmospheric concentration changes, and sequestration where relevant. For a detailed explanation of each result, please see the glossary (Section 6).

Table 23: Climate Impacts

| Scenario | Maximum Annual Emissions Reduction | Total Emissions Reduction |
| --- | --- | --- |
|  | (Gt CO2-eq/yr.) | Gt CO2-eq/yr. (2020-2050) |
| Plausible | 4.83 | 101.28 |
| Drawdown | 5.45 | 108.28 |
| Optimum | 6.12 | 115.28 |

The solution was integrated with all other Project Drawdown solutions and may have different emissions results from the models. This is due to adjustments caused by interactions among solutions that limit full adoption (such as by feedstock or demand limits) or that limit the full benefit of some solutions (such as reduced individual solution impact when technologies are combined).

Table 24: Impacts on Atmospheric Concentrations of CO2-eq

| Scenario | GHG Concentration Change in 2050 | GHG Concentration Rate of Change in 2050 |
| --- | --- | --- |
|  | PPM CO2-eq (2050) | PPM CO2-eq change from 2049-2050 |
| Plausible | 8.35 | 0.35 |
| Drawdown | 8.98 | 0.41 |
| Optimum | 9.61 | 0.47 |

Figure 19: World Annual Greenhouse Gas Emissions Reduction

## Financial Impacts

Below are the financial results of the analysis for each scenario. For a detailed explanation of each result, please see the glossary.

Figure 20: Financial Impacts

| Scenario | Cumulative First Cost | Marginal First Cost | Net Operating Savings | Net Profit Margin | Lifetime Profit Margin | Lifetime Cashflow Savings NPV (of All Implementation Units) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2015-2050 Billion USD | 2015-2050 Billion USD | 2020-2050 Billion USD | 2020-2050 Billion USD | 2020-2050 Billion USD | Billion USD |
| Plausible | $0 | $0 | - $610.67 | - $629.24 | -$178.57 | $0 |
| Drawdown | $0 | $0 | - $610.67 | - $629.24 | -$178.57 | $0 |
| Optimum | $0 | $0 | - $610.67 | - $629.24 | -$178.57 | $0 |

# Discussion

The total emissions avoided from refrigerant management solution has been estimated to be 89.74 Gt CO2eq. from 2020-2050. In 2050, 5.16Gt CO2eq can be avoided from refrigerant destruction, annually. The estimated contribution from the destruction of refrigerants at EOL and from avoiding emissions by adopting good practices are shown in Table 25and Table 26.

Table 25: Estimated destruction of refrigerant (kton)

|  | Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | CFC | HCFC | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2010 | 3.11 | 0.00 | 1.31 | 0.10 | 4.15 | 10.39 | 19.05 |
| 2020 | 34.33 | 1.44 | 23.61 | 2.95 | 9.04 | 48.67 | 120.03 |
| 2030 | 145.00 | 3.72 | 130.33 | 7.40 | 10.45 | 53.24 | 350.13 |
| 2040 | 371.45 | 13.76 | 408.60 | 20.31 | 15.07 | 74.57 | 903.76 |
| 2050 | 456.89 | 14.86 | 502.58 | 29.25 | 0.00 | 37.28 | 1040.86 |

Table 26: Estimated refrigerant emissions avoided by adoption of good practices (kton)

|  | Commercial and industrial | Domestic | Stationary AC (air to air) | Mobile AC | CFC | HCFC | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2010 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| 2020 | 54.78 | 3.66 | 19.29 | 13.45 | 0.00 | 77.31 | 168.50 |
| 2030 | 219.11 | 7.33 | 96.46 | 26.91 | 0.00 | 77.31 | 427.11 |
| 2040 | 492.99 | 21.98 | 260.43 | 60.55 | 0.00 | 92.77 | 928.73 |
| 2050 | 606.38 | 23.74 | 320.33 | 87.19 | 0.00 | 46.39 | 1084.03 |

The estimates show that both parts of the solution contribute almost equally to mitigating emission of refrigerants. But the curve for uptake of both parts of the solutions is very steep as they are a function of the assumptions. The assumptions on the recovery rates can be justified by noting the historical recovery efficiencies in different countries which are presented in Table 27.

Table 27: Recovery efficiencies at EOL for commercial refrigeration systems

|  | In supermarkets (2006) (Source: Clodic & Barrault, 2011) | Commercial (Source: Metz et al., 2005) |
| --- | --- | --- |
| USA, Canada | 70% | 50% (2002); 90%(2015, mitigation sc.) |
| Japan | 80% | 50% (2002); 90%(2015, mitigation sc.) |
| China | 20% | 0% (2002); 25%(2015, mitigation sc.) |
| Brazil | 30% | 0% (2002); 25%(2015, mitigation sc.) |
| EU-15 | 80% | 50% (2002); 90%(2015, mitigation sc.) |
| Other EU | 75% | 50% (2002); 90%(2015, mitigation sc.) |
| India: A5 model | 20% | 0% (2002); 25%(2015, mitigation sc.) |

Destruction rates varied across sectors and were observed to be 60-70% (stationary AC, Japan); 70-80% (chillers, Japan); and 92-97% (industrial) for different years (Metz et al., 2005).  Further, it is reported that 100% destruction  is technically feasible with the implementation of best practices (USEPA 2009). These figures are similar to our assumptions of destruction rates. However, regional figures for recovery and destruction may vary and the uptake depend on the regulations, incentives and strict monitoring and implementation. Hence the assumed rates for recovery and destruction are justified keeping in view the technical feasibility and the recent pressure for lowering global GHG emissions.

Recovering refrigerant gases from the domestic, industrial and transport refrigeration and air conditioning sectors is critical to prevent large volumes of these gases from being vented. Though several developed countries have come up with regulations prohibiting venting, most developing countries do not have such laws and even enforcement in developed countries is a challenge. Due to the Montreal Protocol, countries have been reporting their consumption of refrigerant gases but there is no accounting on quantities of gases banked in existing equipment stock. The data on penetration of recovery and destruction practices is also limited. In the absence of this data, estimating the quantities of these gases being vented annually due to improper disposal practices is a challenge.

In the domestic sector, due to the smaller volumes of gases contained in refrigerators, there is no economic incentive of recovering and destroying gases. However, the stock of domestic refrigerators is huge and many of these would be older than 15 years, indicating that they also continue to hold refrigerants which have high GWP. It is also easier to regulate refrigerant recovery from commercial refrigeration sector specially the large centralized supermarket sub sector that is controlled by a handful of very large companies.

For recovery and destruction to be practiced, economic feasibility of these methods should be improved. Recovery from larger commercial applications in both refrigeration and air conditioning is economically more viable. However, the costs of justifying establishment of destruction facilities will depend on other market forces. For example, creation of a voluntary carbon market focused on generation of carbon credits from refrigerant destruction. Currently three standards provide guidelines for issuing of credits for ODS destruction projects (CCX, Climate Action Reserve and Voluntary Carbon Standard). According to the World Bank (2011), the voluntary carbon market is projected to grow at a rate of 15% per year, which could stimulate demand for credits from refrigerant destruction projects.

Acknowledgement:

As significant part of this solution and report has been built on the earlier work done by Ms Sumedha Malaviya and the authors gratefully acknowledge her work and insights into the solution.

# References

- ASHRAE. 2017. ASHRAE Handbook—Fundamentals. Atlanta: ASHRAE.

- Baumgardner D, de Fatima Andrade M, Klimont Z, Kuylenstierna J, Carvalho SM, Borgford-Parnell N, Mayol-Bracero OL, Melamed M, et al. (2018). Short-lived climate pollutants: Drivers, regional emissions and measurements. In: Integrated Assessment of Short-lived Climate Pollutants in Latin America and the Caribbean: Improving air quality while contributing to climate change mitigation. pp. 18-53 Paris: United Nations Environment Programme (UNEP) and Climate and Clean Air Coalition (CCAC)

- Denis Clodic, & Stephanie Barrault. (2011). 1990 to 2010 Refrigerant inventories for Europe Previsions on banks and emissions from 2006 to 2030 for the European Union- Executive Summary. Retrieved from http://www.epeeglobal.org/epeedocs/internet/docs/ERIE__Armines_study_-_Executive_Summary_-_October_2011_5889.pdf

- Denis Clodic, Stephanie Barrault, & Sabine Saba. (2010). Global inventories of the worldwide fleets of refrigerating and air conditioning equipment in order to determine refrigerant emissions. The 1990 to 2006 updating- Extracts from the final report. MINES Institute, ParisTech. Retrieved from http://www.ces.mines-paristech.fr/Donnees/data06/653-Short-report-complete.pdf

- EIA. (2009). Recovery and destruction of ODS banks- immediate action for global climate protection. Retrieved October 9, 2015, from http://eia-global.org/images/uploads/Recovery_and_Destruction_of_ODS_Banks_V2.pdf

- El-Sayed, A. R., El Morsi, M., & Mahmoud, N. A. (2018). A Review of the Potential Replacements of HCFC/HFCs Using Environment-Friendly Refrigerants. International Journal of Air-Conditioning and Refrigeration, 1830002.

- ICF International. (2008). Analysis on the Recovery of Fluorinated Greenhouse Gases in EU-27 in the Period 2004-2007 and Determination of Options for Further Progress. Retrieved from http://ec.europa.eu/clima/policies/f-gas/docs/gas_recovery_report_en.pdf

- ICF International. (2011). Lifecycle Analysis of High-Global Warming Potential Greenhouse Gas Destruction. The California Air Resources Board. Retrieved from http://www.arb.ca.gov/research/rsc/10-28-11/item7dfr07-330.pdf

- Institute for Governance & Sustainable Development (IGSD), Washington, DC (Available at www.igsd.org/wp- content/uploads/2018/01/EEPrimerDraft.pdf accessed on 04/05/2019

- ISO. (1999). ISO 11650:1999(en), Performance of refrigerant recovery and/or recycling equipment. Retrieved from https://www.iso.org/obp/ui/#iso:std:iso:11650:ed-1:v1:en

- Linde AG. (n.d.). Refrigerant reclaim solutions-Reuse waste refrigerants and protect the environment. Retrieved from http://www.linde-gas.com/internet.global.lindegas.global/en/images/Refrigerant%20Reclaim%20Solutions%20brochure17_108595.pdf

- Metz, B., Solomon, S., Kuijpers, L., Andersen, S. O., Davidson, O., Pons, J., … others. (2005). IPCC-TEAP special report on Safeguarding the Ozone Layer and the Global Climate System: Issues related to hydrofluorocarbons and perfluorocarbons. Retrieved from http://digital.library.unt.edu/ark:/67531/metadc12053/m1/17/

- RTOC. (2011). UNEP 2010 Report of the Refrigeration, Air Conditioning And Heat Pumps Technical Options Committee. Retrieved from http://ozone.unep.org/Assessment_Panels/TEAP/Reports/RTOC/RTOC-Assessment-report-2010.pdf

- UNEP. (2010). Manual for Refrigeration Servicing Technicians. United Nations Environment Programme. Retrieved from http://www.unep.org/pdf/7443-e-Ref_manual_servicing_technicians.pdf

- USEPA, & ADEME. (2004). Determination of comparative HCFC and HFC emission profiles for the Foam and Refrigeration sectors until 2015. Part 1 – Refrigerant emission profiles. Retrieved from http://www3.epa.gov/ozone/snap/emissions/downloads/FoamEmissionProfiles_Part1.pdf

- USEPA. (2009). ODS Destruction in the United States of America and Abroad. Retrieved from https://ozone.unep.org/Data_Reporting/USA-Decision-XX-7-ODS-Destruction.pdf

- World Bank. (2010). Study on Financing the Destruction of Unwanted Ozone-Depleting Substances through the Voluntary Carbon Market- Final Report. Retrieved from http://siteresources.worldbank.org/INTMP/Publications/22587937/FinancingODS_Destruction.pdf

- Zhao, L., Zeng, W., & Yuan, Z. (2015). Reduction of potential greenhouse gas emissions of room air-conditioner refrigerants: a life cycle carbon footprint analysis. Journal of Cleaner Production, 100, 262–268. http://doi.org/10.1016/j.jclepro.2015.03.063

- Devotta, S., Asthana, S., & Joshi, R. (2004). Challenges in recovery and recycling of refrigerants from Indian refrigeration and air-conditioning service sector. Atmospheric Environment, 38(6), 845–854. http://doi.org/10.1016/j.atmosenv.2003.10.039

- TEAP. (2009). TEAP Task Force Decision XX/7 – Phase 2 Report “Environmentally Sound Management of banks Of Ozone-Depleting Substances. Retrieved from http://ozone.unep.org/Assessment_Panels/TEAP/Reports/TEAP_Reports/teap-october-2009-decisionXX-7-task-force-phase2-report.pdf

- RTOC. (2015, February). UNEP 2014 Report of the Refrigeration, Air Conditioning And Heat Pumps Technical Options Committee. Retrieved from http://ozone.unep.org/sites/ozone/files/documents/RTOC-Assessment-Report-2014.pdf

- Velders, G.J.M., Fahey, D.W., Daniel, J.S., Andersen, S.O., and McFarland, M.: Future atmospheric abundances and climate forcings from scenarios of global and regional hydrofluorocarbon (HFC) emissions. Atmospheric Environment, 123, 200-209, doi:10.1016/j.atmosenv.2015.10.071, 2015.

- Velders, G. J. M., Solomon, S., and Daniel, J. S.: Growth of climate change commitments from HFC banks and emissions, Atmos. Chem. Phys., 14, 4563-4572, doi:10.5194/acp-14-4563-2014, 2014.

- Velders, G.J.M., Fahey, D.W., Daniel, J.S., McFarland, M., and Andersen, S.O.: The large contribution of projected HFC emissions to future climate forcing. Proceedings of the National Academy of Sciences 106 (27), 10949–10954, doi: 10.1073/pnas.0902817106, 2009.

- Purohit, P. Höglund-Isaksson, L.:Global emissions of fluorinated greenhouse gases 2005-2050 with abatement potentials and costs. Atmos. Chem. Phys. Discuss., doi:10.5194/acp-2016-727, 2016. Manuscript under review for journal Atmos. Chem. Phys. http://www.atmos-chem-phys-discuss.net/acp-2016-727/acp-2016-727.pdf

- Gschrey, B. and Schwarz, W.: Projections of global emissions of fluorinated greenhouse gases in 2050. Federal Environmental Agency, 2009.

- TEAP. (2009, October). TEAP Task Force Decision XX/7 – Phase 2 Report “Environmentally Sound Management of banks Of Ozone-Depleting Substances. Retrieved from http://ozone.unep.org/Assessment_Panels/TEAP/Reports/TEAP_Reports/teap-october-2009-decisionXX-7-task-force-phase2-report.pdf

- Tohka, A.: The GAINS Model for Greenhouse Gases - Version 1.0: HFC, PFC and SF6. International Institute for Applied Systems Analysis (IIASA), Laxenburg, 2005.

- Zanchi, Vedran, Luka Boban i Vladimir Soldo. "Refrigerant Options in the Near Future." Journal of Sustainable Development of Energy, Water and Environment Systems 7, br. 2 (2019): 293-304.

# Glossary

Adoption Scenario – the predicted annual adoption over the period 2015 to 2060, which is usually measured in Functional Units. A range of scenarios is programmed in the model, but the user may enter her own. Note that the assumption behind most scenarios is one of growth. If for instance a solution is one of reduced heating energy usage due to better insulation, then the solution adoption is translated into an increase in use of insulation. There are two types of adoption scenarios in use: Reference (REF) where global adoption remains mostly constant, and Project Drawdown Scenarios (PDS) which illustrate high growth of the solution.

Approximate PPM Equivalent – the reduction in atmospheric concentration of CO2 (in PPM) that is expected to result if the PDS Scenario occurs. This assumes a discrete avoided pulse model based on the Bern Carbon Cycle model.

Average Abatement Cost – the ratio of the present value of the solution (Net Operating Savings minus Marginal First Costs) and the Total Emissions Reduction. This is a single value for each solution for each PDS Scenario, and is used to build the characteristic “Marginal Abatement Cost” curves when Average Abatement Cost values for each solution are ordered and graphed.

Average Annual Use – the average number of functional units that a single implementation unit typically provides in one year. This is usually a weighted average for all users according to the data available. For instance, total number of passenger-km driven by a hybrid vehicle in a year depends on country and typical number of occupants. We take global weighted averages for this input. This is used to estimate the Replacement Time.

Cumulative First Cost – the total First Cost of solution Implementation Units purchased in the PDS Scenario in the analysis period. The number of solution implementation units that are available to provide emissions reduction during the analysis period is dependent on the units installed prior to the analysis period, and hence all implementation units installed after the base year are included in the cumulative first costing (that is 2015-2050).

Direct Emissions – emissions caused by the operation of the solution, which are typically caused over the lifetime of the solution. They should be entered into the model normalized per functional unit.

Discount Rate- the interest rate used in discounted cash flow (DCF) analysis to determine the present value of future cash flows. The discount rate in DCF analysis takes into account not just the time value of money, but also the risk or uncertainty of future cash flows; the greater the uncertainty of future cash flows, the higher the discount rate.   Most importantly, the greater the discount rate, the more the future savings are devalued (which impacts the financial but not the climate impacts of the solution).

Emissions Factor– the average normalized emissions resulting from consumption of a unit of electricity across the global grid. Typical units are kg CO2e/kWh.

First Cost- the investment cost per Implementation Unit which is essentially the full cost of establishing or implementing the solution.  This value, measured in 2014$US, is only accurate to the extent that the cost-based analysis is accurate.  The financial model assumes that the first cost is made entirely in the first year of establishment and none thereafter (that is, no amortization is included).  Thus, both the first cost and operating cost are factored in the financial model for the first year of implementation, all years thereafter simply reflect the operating cost until replacement of the solution at its end of life.

Functional Unit – a measurement unit that represents the value, provided to the world, of the function that the solution performs. This depends on the solution. Therefore, LED Lighting provides petalumen-hours of light, Biomass provides tera-watt-hours of electricity and high speed rail provides billions of passenger-km of mobility.

Grid Emissions – emissions caused by use of the electricity grid in supplying power to any operation associated with a solution. They should be in the units described below each variable entry cell. Drawdown models assume that the global electric grid, even in a Reference Scenario, is slowly getting cleaner, and that emissions factors fall over time resulting in lower grid emissions for the same electricity demand.

Implementation Unit – a measurement unit that represents how the solution practice or technology will be installed/setup and priced. The implementation unit depends on the solution. For instance, implementing electric vehicles (EV) is measured according to the number of actual EV’s in use, and adoption of Onshore Wind power is measured according to the total terawatts (TW) of capacity installed worldwide.

Indirect Emissions – emissions caused by the production or delivery or setup or establishment of the solution in a specified area. These are NOT caused by day to day operations or growth over time, but they should be entered into the model normalized on a per functional unit or per implementation unit basis.

Learning Rate/Learning Curve - Learning curves (sometimes called experience curves) are used to analyze a well-known and easily observed phenomenon: humans become increasingly efficient with experience. The first time a product is manufactured, or a service provided, costs are high, work is inefficient, quality is marginal, and time is wasted. As experienced is acquired, costs decline, efficiency and quality improve, and waste is reduced. The model has a tool for calculating how costs change due to learning. A 2% learning rate means that the cost of producing a good drops by 2% every time total production doubles.

Lifetime Capacity – this is the total average functional units that one implementation unit of the solution or conventional technology or practice can provide before replacement is needed. All technologies have an average lifetime usage potential, even considering regular maintenance. This is used to estimate the Replacement Time. and has a direct impact on the cost to install/acquire technologies/practices over time. E.g. solar panels generate, on average, a limited amount of electricity (in TWh) per installed capacity (in TW) before a new solar panel must be purchased. Electric vehicles can travel a limited number of passenger kilometers over its lifetime before needing to be replaced.

Lifetime Operating Savings–the operating cost in the PDS versus the REF scenarios over the lifetime of the implementation units purchased during the model period regardless of when their useful life ends.

Lifetime Cashflow NPV-the present value (PV) of the net cash flows (PDS versus REF) in each year of the model period (2015-2060). The net cash flows include net operating costs and first costs. There are two results in the model: Lifetime Cashflow NPV for a Single Implementation Unit, which refers to the installation of one Implementation Unit, and Lifetime Cashflow NPV of All Units, which refers to all Implementation Units installed in a particular scenario. These calculations are also available using profit inputs instead of operating costs.

Marginal First Cost – the difference between the First Cost of all units (solution and conventional) installed in the PDS Scenario and the First Cost of all units installed in the REF Scenario during the analysis period. No discounting is performed. The number of solution implementation units that are available to provide emissions reduction during the analysis period is dependent on the units installed prior to the analysis period, and hence all implementation units installed after the base year are included in the cumulative first costing (that is 2015-2050).

Net Annual Functional Units (NAFU) – the adoption in the PDS minus the adoption in the REF in each year of analysis. In the model, this represents the additional annual functional demand captured either by the solution in the PDS Scenario or the conventional in the REF Scenario.

Net Annual Implementation Units (NAIU) – the number of Implementation Units of the solution that are needed in the PDS to supply the Net Annual Functional Units (NAFU). This equals the adoption in the PDS minus the adoption in the REF in each year of analysis divided by the average annual use.

Net Operating Savings – The undiscounted difference between the operating cost of all units (solution and conventional) in the PDS Scenario minus that of all units in the REF Scenario.

Operating Costs – the average cost to ensure operation of an activity (conventional or solution) which is measured in 2014$US/Functional Unit.  This is needed to estimate how much it would cost to achieve the adoption projected when compared to the REF Case. Note that this excludes First Costs for implementing the solution.

Payback Period – the number of years required to pay all the First Costs of the solution using Net Operating Savings. There are four specific metrics each with one of Marginal First Costs or First Costs of the solution only combined with either discounted or non-discounted values. All four are in the model. Additionally, the four outputs are calculated using the increased profit estimation instead of Net Operating Savings.

PDS/ Project Drawdown Scenario – this is the high growth scenario for adoption of the solution

PPB/ Parts per Billion – a measure of concentration for atmospheric gases. 10 million PPB = 1%.

PPM/ Parts per Million – a measure of concentration for atmospheric gases. 10 thousand PPM = 1%.

REF/ Reference Scenario – this is the low growth scenario for adoption of the solution against which all PDS scenarios are compared.

Regrets solution has a positive impact on overall carbon emissions being therefore considered in some scenarios; however, the social and environmental costs could be harmful and high.

Replacement Time- the length of time in years, from installation/acquisition/setup of the solution through usage until a new installation/acquisition/setup is required to replace the earlier one.  This is calculated as the ratio of Lifetime Capacity and the Average Annual Use.

TAM/ Total Addressable Market – represents the total potential market of functional demand provided by the technologies and practices under investigation, adjusting for estimated economic and population growth. For this solutions sector, it represents world and regional total addressable markets for electricity generation technologies in which the solutions are considered.

Total Emissions Reduction – the sum of grid, fuel, indirect, and other direct emissions reductions over the analysis period. The emissions reduction of each of these is the difference between the emissions that would have resulted in the REF Scenario (from both solution and conventional) and the emissions that would result in the PDS Scenario. These may also be considered as “emissions avoided” as they may have occurred in the REF Scenario, but not in the PDS Scenario.

Transition solutions are considered till better technologies and less impactful are more cost effective and mature.

TWh/ Terawatt-hour – A unit of energy equal to 1 billion kilowatt-hours

Halocarbons are compounds formed from methane (CH4) and ethane (C2H6) molecules by replacement of the hydrogen atoms with halogens like chlorine, flouring and bromine.

Consumption= (Production + imports)-exports

Bahrain, India, Iran, Iraq, Kuwait, Oman, Pakistan, Qatar, Saudi Arabia, and UAE

http://www.epa.gov/rad/environmental-benefits

https://refrigerantreclaim.com.au/our-role/ozone-protection-act/

In some cases, the low boundary is negative for a variable that can only be positive, and in these cases the lowest collected data point is used as the “low” boundary.