# Project Drawdown

# Integration of Transportation Sector Solution Models

Contents

Introduction	2

Modeled Solution Definitions	2

Solution Classification	3

Solution Modeling	5

Key Model Assumptions	6

Adoption Scenarios	6

Integration	7

Market/TAM Consistency	7

Variable Consistency	8

Grid Demand Changes	9

Other Interactions not Directly Modeled	9

Final Results	10

Appendices	10

Appendix 1 – Definitions	10

Appendix 2 – Key Variables	13

# Introduction

Drawdown is the point in time when atmospheric greenhouse gas concentrations begin to decline on a year to year basis. This is only possible with significant adoption of replacement, reduction, and biosequestration technologies and practices. Numerous solutions to global warming already exist, and innovative technologies are being developed which could have large impact on greenhouse gas concentrations in the future. Project Drawdown is developing models to examine the emissions and financial impact of ambitious but plausible adoption of climate solutions that already exist. The goal is to see what it would take to get to drawdown with these technologies and practices that are viable, scaling, and financially feasible. Starting with the existing solutions necessitates the use of a bottom-up approach in modeling the cumulative impact. It is critical that individual models are integrated to ensure that no double-counting occurs, and that no unrealistic relationships are implied. This document details the integration of Project Drawdown’s solution models after briefly introducing the modeling process and solutions models themselves.

Several of our transportation solutions have published projections of adoption, variables and supporting technologies that affect other solutions within or outside of the sector. We have attempted to account for the most impactful of these relationships, and we describe how in this document. The relationships depend heavily on the definition and scope of each solution, and so we included a description or delimitation of the included solutions first. Note that some solutions, as included in Project Drawdown, may not be as broad as all that the title may imply, and so the reader is strongly encouraged to read the detailed definitions in Appendix 1 – Definitions before examining integration. This restriction in scope for some solutions was performed to limit the solution to what could be modeled in a reasonable amount of time, and with publicly-available data. However, our technical reports review a much broader interpretation of the solutions. We provide later in this document, an overview of the relationships among the solutions. Finally, we describe the steps taken to account for double counting and interaction impacts.

# Modeled Solution Definitions

Included in the Project Drawdown list are 13 of the most impactful solutions for reducing emissions in the transport sector. The list excludes some solutions that are impactful, but future work of Project Drawdown will include as many other solutions as possible.

- Efficient Aviation – increased use of technologies to reduce aircraft fuel burn

- Bicycle Infrastructure – increased installation of bike paths to encourage more bike usage (placed with Buildings and Cities in Drawdown)

- Hybrid Cars – increased use of hybrid cars

- Carpooling – increased ride-sharing of all types that generally increase average passenger load factors (including ridesharing app usage)

- Electric Bicycles – increased use of electric bikes instead of cars for urban travel

- Electric Cars – increased use of battery electric and plug-in hybrid vehicles

- High-speed Rail – track construction for increased use of high speed rail for intercity passenger travel

- Public Transit – increased usage of mass transit or public transport to get around cities

- Efficient Ocean Shipping – the use of technologies to make maritime shipping less fuel-intensive

- Telepresence – replacing flying for business meetings with telepresence meetings

- Electric Trains – increased electrification of freight railways

- Efficient Trucks – increased use of fuel reduction technologies and approaches for trucking

- Walkable Cities – increased walking instead of driving for urban travel (placed with Buildings and Cities in Drawdown)

# Solution Classification

We illustrate the main groupings and relationships among solutions based on the definitions of each in Figure 1.

Figure 1 Project Drawdown Transport Solution Classification by Mobility Type, Model Units, and Solution Type

There are many groupings shown in Figure 1: the mobility cluster as columns, the units used in the modelling as rows, and the type of solution according to the Avoid-Shift-Improve (ASI) framework in outline. Here, “functional units” are measures of the value provided by the solution to the society: mobility of people or mobility of freight. “Implementation units” measure the actual implementation of the solution, or what we actually pay for when implementing.

Each mobility cluster was defined using several sources, including the International Energy Agency (IEA), the International Council for Clean Transportation (ICCT), the Institute for Transportation and Development Policy (ITDP), and the University of California at Davis (UCD). All urban passenger transport solutions, for instance, share the urban transport market; thus, the adoption of each is related to the adoption of others. Note that we do not consider air freight, we do, however, consider the use of electric vehicles and hybrids in the urban and non-urban realms by ensuring market and adoption consistency.

Solution types were used to identify similar elements among the solutions, and is mainly for communication of the solutions, however this was useful for prioritizing solutions for climate impact. The ASI Framework, as simple as it may seem, helps simplify the prioritization process by separating solutions that have the least direct carbon emissions from those that have merely lower emissions than conventional options. The prioritization generally follows the A->S->I groups as shown in Figure 1 (with “Avoid” type solutions having the highest priority).

The units used for each solution were selected to enable easy modelling and data collection as well as easy integration with other solutions. Functional units were generally consistent: passenger-km (or billions of passenger-km) for passenger solutions, and million ton-km for freight solutions. Only Ship Fuel Efficiency used billion ton-nautical mile to be consistent with the maritime sector and its data sources. Most implementation units are self-explanatory, but one requires brief explanation: Video-conference User for Telepresence. Since Telepresence systems come is a variety of technologies with a variety of number of users, this unit was found to provide the flexibility needed to capture all the technologies, although not the quality of the communication that they provide. A Skype system for instance would be considered to have 1 “video-conference user” whereas a single immersive telepresence system may have 6. The call quality and implementation costs still vary widely between those two examples however, and to capture this, weighting with estimated market shares was used.

The Transport Sector solutions clusters are described below:

- Urban Passenger Transport – the movement of people within built-up areas, which are often not fully within the administrative limits of a city. In reality, many cities’ metropolitan areas extend well outside of the city, and the movement of people within this entire area is considered urban transport by many in transport policy and research. All solutions in this cluster use passenger-kilometers as their units.

- Non-Urban Passenger Transport – the movement of people either completely outside of urban areas (that is, rural transport), or between urban areas. Three facts motivate the interpretation of travel in this cluster as chiefly between urban areas: i) more than half of the world’s population already lives in urban areas, with around 50% of all travel happening completely within cities; ii) urban populations are growing faster than non-urban population; and iii) long-distance travel (travel of over 50km, which is around 40% of all travel) is made more often by wealthier people who live in cities.  We therefore use the terms “non-urban” and “intercity” for this cluster interchangeably. All solutions in this cluster use passenger-kilometers as their units.

- Freight Transport – the movement of goods anywhere by any means. The ships solution uses billion ton-nautical miles as its units for consistency with the industry, but the other two solutions use million ton-kilometers.

# Solution Modeling

The models developed by Project Drawdown each examine the emissions and financial effect of ambitious adoption of a specific solution to global warming from 2020-2050. Solutions are defined based on an assessment of current literature. Typically, this is based on boundaries that are well understood by those researching its application and implementation. In some cases, though the boundaries are not clearly defined, or the solution represents a wide range of actual methods of implementation. This led to the definition of solutions based on access to data sources and their applicability with the model framework. The scope of individual solutions as modeled by the Project Drawdown research team may be limited to specific technologies and practices (e.g. High Efficiency Heat Pumps or Silvopasture), or a combination of several (e.g. Oceanic Freight or Managed Grazing). Definitions may differ from other modeling efforts.

Individual solutions were modeled separately guided by common datasets that were developed for the sector (e.g. conventional car prices). These models were then integrated in an iterative process that is illustrated in Figure 2. Technical reports for each model are available from Project Drawdown, and the integration process is described in the Integration Section of this document.

Each model compares a clearly defined and delimited conventional technology or practice to the solution. Data on conventional technologies/practices and solutions are entered into the models, and boundaries created using statistical analysis. This allows users to make reasonable, data-driven choices for all inputs.  All non-land solution and conventional adoption is assumed bounded by the total addressable market (TAM) which is specifically defined for each solution but may be shared across several competing solutions (such as all the urban passenger transport solutions and conventional technologies using the same TAM).

## Key Model Assumptions

The assumptions applied across all Project Drawdown scenarios were:

- Infrastructure required to support solution is already in place

- Policies to encourage adoption are already in place

- A carbon price is not necessarily assumed

- Costs and savings are at agency level

- Prices may change according to learning

- Included solutions may change significantly in future, but this is not considered

## Adoption Scenarios

Three general Project Drawdown scenarios were developed for the Transport Sector. These were implemented in each solution model, and in some cases were updated during the integration process:

- Plausible Scenario: this scenario represents incremental but optimistic and realistic adoption of the solution to 2050.

- Drawdown Scenario:  this is a scenario aligned to the world achieving drawdown by 2050.

- Optimum Scenario:  this represents what the modeling team saw as the maximum potential for a solution’s adoption or impact.

Figure 2 Project Drawdown Transport Sector Model Relations

# Integration

To integrate all the transportation solutions, we have three main components:

- Market/TAM/Adoption Consistency: Ensuring that all solutions that are in the same market:

  - use the same TAM data,

  - use consistent market shares, and

  - have projected adoptions that do not exceed the total projected demand

- Variable Consistency: Ensuring that all variables that are used in several solutions have the same values.

- Grid Demand Changes: Ensuring that the effects of transportation electrification on the grid are accounted for and vice versa.

Figure 3 shows the integration process.

Figure 3 Transport Integration Process

## Market/TAM/Adoption Consistency

TAM or Total Transport Demand Data and Consistent Market shares– we have collected projections of passenger and freight travel from several sources and combined them for use across all models. We have also calculated market share/modeshares for all solutions from this combined dataset and use these values in all models. In some cases the sources for modeshare and TAM projections excluded some modes of transport, most notably non-motorized modes like walking and cycling, hence our combined projections have added in this missing portion. Since we also are modeling Telepresence, which avoids transport altogether, we have estimated the equivalent current mobility replaced by Telepresence (only for non-urban passenger transport) and added it to the TAM for non-urban transport. What this means is that the TAM’s listed from certain sources like “International Energy Agency (IEA), 2016 – Energy Technology Perspectives 2016” do not match their published data since their original data excluded non-motorized modes, and avoided mobility.

Projected Adoptions – the total adoption for each technology often comes from an independent source, and when summed across all technologies, we found that the totals exceeded the total projection of demand in some cases. We therefore prioritized the modes of transport by their energy use, space efficiency and potential impact on energy use within each cluster. The adoptions of high priority solutions were not reduced, but those of other solutions were reduced. The prioritized order of solutions is as represented in Figure 3.

The adoptions of Electric Cars and Hybrid Cars were adjusted to prevent exceeding the TAM. We have classified them as both urban and  non-urban modes, so the TAM of each of these realms was balanced. The balance in their usage between the two realms was dependent on the split in car usage between urban and non-urban travel. We assumed that hybrid cars have a 50:50 split in usage in the urban and interurban realms, and that it remains constant throughout the analysis period (except in the Drawdown and Optimum Scenarios, where the urban share changes from 2030). For Electric vehicles, we assume that “range anxiety” limits intercity travel to close to 0% until 2020, when battery capacity and infrastructure develops enough to encourage EV users to drive outside of cities in significant amounts. After 2020, the intercity share of EV travel rises by different amounts in each scenario. These splits are not needed for the emissions impact estimates, but are mainly performed to ensure that no exaggeration of EV or hybrid use occurs via overallocation of adoption. Note that these Optimum scenarios would mean much more use of more environmentally-sound modes in urban areas and use of less space-efficient modes (Electric Cars, Hybrid Cars) mainly outside of cities.

## Variable Consistency

For several variables, especially those relating to ICE cars, and discount rates, consistency across solutions was maintained by ensuring that the same values were used in different models needing those variables. The list of variables that were made consistent is:

- Car variables

  - First costs

  - Variable operating costs (maintenance)

  - Fixed operating costs (insurance)

  - Average annual use

  - Load factor

  - Lifetime before replacement

  - Fuel intensity (in gasoline equivalents)

  - Indirect emissions

- Discount Rates (across all of Drawdown models)

- Fuel prices (across all of Drawdown models)

- Fuel Emissions factors (across all of Drawdown models)

- Total Addressable Market/Transportation Demand

- Current modeshares/marketshares

## Grid Demand Changes

To ensure that the increased demand can be met for solutions calling for more grid electricity usage (high speed rail, electric vehicles, electric bikes, train fuel efficiency, mass transit, and telepresence), we summed the net increased grid electricity demand (after reducing those solutions that lowered demand), and verified that the increased demand (including that of other sectors) can be provided by the expanded grid.

## Other Interactions not Directly Modeled

We acknowledge that there are other relationships among the chosen solutions as well as among those solutions and those in other sectors, and we have not modeled them due to their complex nature, lack of scientific consensus on how they work, lack of global data or lack of significance. These include

- The relationship between bike usage, mass transit and increased walking

- The future impact on EV and hybrid use of autonomous cars.

- Potential mode shift from freight truck to freight rail

- Autonomous mass transit

- Urban telepresence

- Air freight emissions reduction from Airplane Fuel Efficiency

- Intercity mass transit combined with urban mass transit (intermodality)

- Cross-modal efficiency technology sharing (airplane, train, truck, oceanic freight, car fuel efficiency) – e.g. improvements in battery technology for cars impacting electric bikes

- Demand rebound/induced demand

- Transportation costs for global food supply

- The relationship between increased active mode use (walking, biking) and increased food consumption and emissions

- Impact of improved refrigerant management and low GWP refrigerants on emissions of transport systems (including leakage from transportation vehicles

- The design of lighter vehicles to improve efficiency

# Final Results

After updating the inputs of the individual solution models to account for integration, the scenarios are resaved- and the new results are considered integrated and final. These are the results that are available in the solution models.

# Appendices

## Appendix 1 – Definitions

- Efficient Aviation (Airplane Fuel Efficiency)

  - The use of technologies and approaches to reduce the amount of fuel needed to fly passenger aircraft. Considering that much air freight travels in the belly of passenger aircraft, this also affects air freight, but is not separately considered. We examine the adoption of the newest generation of aircraft in the single aisle and twin aisle families, or retrofitting aircraft with technologies capable to providing the same benefits.

- Bicycle Infrastructure

  - Implementing bike lanes of various types in order to encourage more bicycle use.

- Hybrid Cars (Car Fuel Efficiency, Hybrids)

  - The use of any of several car technologies to reduce the amount of fuel needed to power cars. Of all the technologies reviewed, hybrid cars appear to be the most promising, and so our modelling focusses on those. “Hybrid” here means cars that have two power trains (one for the gasoline, and one for electrical energy). However these cars cannot be plugged in for recharging, all recharging comes from the movement of the internal combustion engine. We consider its impact on urban and non-urban passenger travel.

- Carpooling (all types that lead to higher load factors)

  - The sharing of car trips (as opposed to the sharing of vehicles when not in use = car sharing) by adding more passengers in order to reduce individual or societal costs per traveler. The term Ridesharing implies many concepts. Here we consider all interpretations and implementations that increase vehicle occupancy for any car-based mobility. This therefore includes Uber and Lyft app impacts, promotion of carpooling, use of car pool/high occupancy vehicle lanes, commuter ridesharing programs, increased cultural acceptance of ridesharing, and even impacts of road pricing (congestion pricing). However only user prices are included: the trip prices are divided by total travelers and total distance traveled to get the per passenger-km price.

- Electric Bicycles

  - Bicycles that are at least partly powered by rechargeable batteries.

- Electric Cars

  - Cars that are at least partly powered by an electric drive train with batteries that can be charged by plugging-in. We classify this mainly as an urban mode, but do consider its impact on intercity passenger travel.

- High Speed Rail (HSR)

  - High Speed Passenger rail systems as defined by the International Union of Railways (UIC). We focus on the construction of the infrastructure that would lead to greater HSR travel, but we also model the impact of higher usage of existing rails.

- Public Transit

  - The use of urban mass transit systems for passenger travel, including metro, tram/trolley, light rail transit, bus, bus rapid transit, and minivan. We focus on the passenger taking more mass transit trips, but do not model the increase in infrastructure or public investment that may be required to supply all those trips.

- Efficient Ocean Shipping (Ship Fuel Efficiency, Oceanic Freight Improvements)

  - Any of several technologies that can reduce the use of ship fuel for moving maritime ships. We focus on international shipping, which is the vast majority of shipping globally, and we limit the problem to four types of vessels above 1000 gross tons: bulk carriers, oil tankers, container ships and general cargo ships, together accounting for the majority of all transport work done internationally.

- Telepresence (Business Travel Avoidance)

  - Any digital technology than can allow (at least) visual and auditory communication with others far away. The largest parts of this market are provided with software-only tools (such as Zoom or Skype), but also with hardware based systems like immersive rooms containing large screens and surround microphone and speaker systems. In the future, virtual- or augmented- reality systems are likely, and the impact on urban, intercity, business and leisure/family travel should be significant. We focus on the potential market currently provided by software and mid-range hardware systems, and on the intercity business travel market.

- Electric Trains (Train Fuel Efficiency)

  - Any of several technologies that can reduce the energy needed to move freight trains. Of those identified in the literature, electrification of trains is noted as the most effective, hence we focus on that.

- Efficient Trucks (Truck Fuel Efficiency)

  - A technology or approach than can reduce fuel usage for freight truck power. The range of impactful technologies is great, and so we have modeled adoption of a package of the most commonly cited technologies which include hybridization of trucks, light-weighting and low resistance tires.

- Walkable Cities

  - These are city areas where walking can be high due to a range of city-specific (as opposed to culture-specific) factors such as the “seven D’s” – design of streets, density of the city areas, diversity of land uses, distance to transit, demand management, destination accessibility and the demographics of the residents. The research on these seven factors is limited, and only density has been studied at length, hence we use this as a metric for how walking can be increased worldwide over time.

## Appendix 2 – Key Variables

For transparency, key variables used in the modeling process have been extracted and described in Table 1.

Table 1 Key Variables used in the Transport Sector Modeling

| # | Variable Name | Definition | Solutions Using Variable | Value | Units | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | ICE Car price | Average purchase price of new internal combustion engine (ICE) light duty vehicle (LDV) | Cars, E-bikes, EV, Mass Transit, Walkable Cities. | $27,302 | 2014$US | Segment-weighted, Country-weighted, EIA data. |
| 2 | ICE Car fuel intensity | Average amount of fuel used when driving | Bike Infrastructure, Cars, E-bikes, EV, Mass Transit, Ridesharing, Walkable Cities. | 0.05 | Liters gasoline equivalent per passenger-km | Averaged from 19 data-points |
| 3 | ICE Car lifetime | The average total mobility that one LDV provides before being retired or scrapped | Cars, E-bikes, EV, Mass Transit, Ridesharing, Walkable Cities. | 198,000 | Vehicle-km | Averaged from 12 data-points |
| 4 | ICE Car indirect emissions | The average amount of emissions produced in production and delivery of an ICE LDV | Cars, E-bikes, EV, Mass Transit, Walkable Cities. | 9.7 | Metric tons of CO2-eq per car | Averaged from 20 data-points |
| 5 | Average car occupancy | The average number of people in any car trip globally | Cars, E-bikes, EV, Mass Transit, Ridesharing, Walkable Cities. | 1.57 | Persons per car | Average of several countries’ data |
| 6 | Gasoline price | Average global petrol price from 2005 - 2014 | Cars, E-bikes, EV, Mass Transit, Ridesharing, Walkable Cities. | $1.04 | 2014$US/ liter | From International Energy Agency |
| 7 | Diesel price | Average global diesel price from 2005 - 2014 | Trains, Trucks. | $1.31 | 2014$US/ liter | From International Energy Agency |
| 8 | Battery price | Average global unit price of installing battery capacity | E-bikes. | $391 | 2014$US/ kWh of installed capacity | This is for e-bike batteries only. |
| 9 | Consumer Electricity price | Average global electricity supply price for consumers | E-bikes, EV. | $0.14 | 2014$US/ kWh supplied | Country-weighted average of 509 estimates |
| 10 | Commercial Electricity price | Average global electricity supply price for commercial entities | HSR, Mass Transit, Telepresence, Trains. | $0.09 | 2014$US/ kWh supplied | Country-weighted average of 838 estimates |
| 11 | Public Sector Discount Rate | Time cost of money for all public entities | Bike Infrastructure, HSR. | 6.38% | Percent | Averaged from 20 data-points |
| 12 | Consumer Discount Rate | Time cost of money for all private individuals | Cars, E-bikes, EV, Mass Transit, Ridesharing, Walkable Cities. | 4.0% | Percent | Averaged from 6 data-points |
| 13 | Commercial Discount Rate | Time cost of money for all private companies | Airplanes, Ships, Telepresence, Trains, Trucks. | 9.22% | Percent | Averaged from 5 data-points |

The ASI framework, originally developed in Germany, seeks to focus on managing demand for mobility while minimizing externalities, particularly climate impacts. See www.sutp.org

See Hayashi, Y., Matsuoka, I., Fujisaki, K., Itoh, R., Kato, H., Rothengatter, W., & Takeshita, H. (2015). Importance of Intercity Passenger Transport for Climate Change Issues. In Intercity Transport and Climate Change (pp. 1-30). Springer, Cham.

For most results, only the differences between scenarios, summed over 2020-2050 were presented, but for the net first cost, the position was taken that to achieve the adoptions in 2020, growth must first happen from 2015 to 2020, and that growth comes at a cost which should be accounted for, hence net first cost results represent the period 2015-2050.

Range anxiety is the fear of not having sufficient battery power or charging stations to complete a long trip by electric vehicle. This would be akin to early days of petrol car use when gas stations were much rarer than today, and long distance trips were harder.

GWP – Global Warming Potential – the average global warming impact of one kg of a greenhouse gas compared to one kg of Carbon Dioxide/CO2. Refrigerants typically have GWP over 2,000.

The US Energy Information Administration